ПРИЛОЖЕНИЕ Е

(обязательное)

Полный VHDL-код проекта

В приложение вынесены исходные VHDL-модули микро-ЭВМ и проверочные стенды. Код не включён в основной текст пояснительной записки, чтобы не нарушать структуру ПЗ.

Е.1 Исходные VHDL-модули проекта

alu_core.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity alu_core is

port (

a_i      : in  word_t;

b_i      : in  word_t;

flag_s_i : in  std_logic;

op_i     : in  std_logic_vector(2 downto 0);

y_o      : out word_t;

z_o      : out std_logic;

s_o      : out std_logic;

c_o      : out std_logic;

o_o      : out std_logic

);

end entity;

architecture rtl of alu_core is

begin

process(a_i, b_i, flag_s_i, op_i)

variable a_u   : unsigned(15 downto 0);

variable sum17 : unsigned(16 downto 0);

variable y_v   : word_t;

variable z_v   : std_logic;

variable s_v   : std_logic;

variable c_v   : std_logic;

variable o_v   : std_logic;

begin

a_u := unsigned(a_i);

y_v := a_i;

c_v := '0';

o_v := '0';

case op_i is

when ALU_OR =>

y_v := a_i or b_i;

when ALU_NOR =>

y_v := not (a_i or b_i);

when ALU_SRA =>

y_v := a_i(15) & a_i(15 downto 1);

c_v := a_i(0);

when ALU_INCS =>

sum17 := ('0' & a_u);

if flag_s_i = '1' then

sum17 := ('0' & a_u) + to_unsigned(1, 17);

end if;

y_v := std_logic_vector(sum17(15 downto 0));

c_v := sum17(16);

o_v := (not a_i(15)) and flag_s_i and y_v(15);

when others =>

y_v := a_i;

end case;

if y_v = x"0000" then

z_v := '1';

else

z_v := '0';

end if;

s_v := y_v(15);

y_o <= y_v;

z_o <= z_v;

s_o <= s_v;

c_o <= c_v;

o_o <= o_v;

end process;

end architecture;

branch_predictor.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity branch_predictor is

port (

clk_i          : in  std_logic;

rst_i          : in  std_logic;

query_valid_i  : in  std_logic;

pc_query_i     : in  addr_t;

hist_o         : out std_logic_vector(1 downto 0);

pred_taken_o   : out std_logic;

pred_target_o  : out addr_t;

update_valid_i : in  std_logic;

pc_update_i    : in  addr_t;

hist_i         : in  std_logic_vector(1 downto 0);

actual_taken_i : in  std_logic;

actual_tgt_i   : in  addr_t

);

end entity;

architecture rtl of branch_predictor is

type pht_t   is array (0 to 15) of unsigned(1 downto 0);

type target_t is array (0 to 15) of addr_t;

type valid_t  is array (0 to 15) of std_logic;

signal pht       : pht_t    := (others => "01");

signal btb_tgt   : target_t := (others => (others => '0'));

signal btb_valid : valid_t  := (others => '0');

signal ghr_r     : std_logic_vector(1 downto 0) := "00";

function idx_of(pc : addr_t; hist : std_logic_vector(1 downto 0)) return integer is

variable idx_v : std_logic_vector(3 downto 0);

begin

idx_v := std_logic_vector(pc(1 downto 0)) & hist;

return to_integer(unsigned(idx_v));

end function;

function sat_dec(v : unsigned(1 downto 0)) return unsigned is

begin

if v = "00" then

return v;

else

return v - 1;

end if;

end function;

begin

hist_o <= ghr_r;

process(query_valid_i, pc_query_i, ghr_r, pht, btb_valid, btb_tgt)

variable idx : integer;

begin

pred_taken_o  <= '0';

pred_target_o <= (others => '0');

if query_valid_i = '1' then

idx := idx_of(pc_query_i, ghr_r);

if pht(idx)(1) = '1' and btb_valid(idx) = '1' then

pred_taken_o  <= '1';

pred_target_o <= btb_tgt(idx);

end if;

end if;

end process;

process(clk_i)

variable idx : integer;

begin

if rising_edge(clk_i) then

if rst_i = '1' then

pht       <= (others => "01");

btb_tgt   <= (others => (others => '0'));

btb_valid <= (others => '0');

ghr_r     <= "00";

elsif update_valid_i = '1' then

idx := idx_of(pc_update_i, hist_i);

if actual_taken_i = '1' then

pht(idx)       <= sat_inc2(pht(idx));

btb_tgt(idx)   <= actual_tgt_i;

btb_valid(idx) <= '1';

else

pht(idx) <= sat_dec(pht(idx));

end if;

ghr_r <= ghr_r(0) & actual_taken_i;

end if;

end if;

end process;

end architecture;

bus_arbiter_2master.vhd

library ieee;

use ieee.std_logic_1164.all;

entity bus_arbiter_2master is

port (

req_cpu_i   : in  std_logic;

req_dma_i   : in  std_logic;

grant_cpu_o : out std_logic;

grant_dma_o : out std_logic

);

end entity;

architecture rtl of bus_arbiter_2master is

begin

grant_dma_o <= req_dma_i;

grant_cpu_o <= req_cpu_i and (not req_dma_i);

end architecture;

cache_4way_age.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_4way_age is

port (

clk_i        : in  std_logic;

rst_i        : in  std_logic;

cpu_req_i    : in  std_logic;

cpu_we_i     : in  std_logic;

cpu_addr_i   : in  addr_t;

cpu_wdata_i  : in  word_t;

cpu_rdata_o  : out word_t;

cpu_ready_o  : out std_logic;

hit_o        : out std_logic;

miss_o       : out std_logic;

ram_req_o    : out std_logic;

ram_we_o     : out std_logic;

ram_addr_o   : out addr_t;

ram_wdata_o  : out word_t;

ram_rdata_i  : in  word_t;

ram_grant_i  : in  std_logic

);

end entity;

architecture rtl of cache_4way_age is

subtype cache_tag_t is std_logic_vector(11 downto 0);

type cache_state_t is (IDLE, WAIT_READ_GRANT, WAIT_READ_DATA, WAIT_WRITE_GRANT);

type set_way_word_t is array (0 to CACHE_SETS-1, 0 to CACHE_WAYS-1) of word_t;

type set_way_tag_t is array (0 to CACHE_SETS-1, 0 to CACHE_WAYS-1) of cache_tag_t;

type set_way_age_t is array (0 to CACHE_SETS-1, 0 to CACHE_WAYS-1) of unsigned(1 downto 0);

type set_way_valid_t is array (0 to CACHE_SETS-1, 0 to CACHE_WAYS-1) of std_logic;

signal state_r      : cache_state_t := IDLE;

signal data_r       : set_way_word_t := (others => (others => (others => '0')));

signal tag_r        : set_way_tag_t := (others => (others => (others => '0')));

signal age_r        : set_way_age_t := (others => (others => (others => '0')));

signal valid_r      : set_way_valid_t := (others => (others => '0'));

signal req_addr_r   : addr_t := (others => '0');

signal req_wdata_r  : word_t := (others => '0');

signal req_we_r     : std_logic := '0';

signal write_hit_r  : std_logic := '0';

signal victim_r     : integer range 0 to CACHE_WAYS-1 := 0;

signal set_r        : integer range 0 to CACHE_SETS-1 := 0;

signal tag_req_r    : cache_tag_t := (others => '0');

signal cpu_rdata_r  : word_t := (others => '0');

signal cpu_ready_r  : std_logic := '0';

signal hit_r        : std_logic := '0';

signal miss_r       : std_logic := '0';

signal ram_req_r    : std_logic := '0';

signal ram_we_r     : std_logic := '0';

signal ram_addr_r   : addr_t := (others => '0');

signal ram_wdata_r  : word_t := (others => '0');

function cache_set(addr : addr_t) return integer is

begin

return to_integer(unsigned(addr(3 downto 0)));

end function;

function cache_tag(addr : addr_t) return cache_tag_t is

begin

return addr(15 downto 4);

end function;

function inc_age(v : unsigned(1 downto 0)) return unsigned is

begin

if v = "11" then

return v;

else

return v + 1;

end if;

end function;

begin

cpu_rdata_o <= cpu_rdata_r;

cpu_ready_o <= cpu_ready_r;

hit_o       <= hit_r;

miss_o      <= miss_r;

ram_req_o   <= ram_req_r;

ram_we_o    <= ram_we_r;

ram_addr_o  <= ram_addr_r;

ram_wdata_o <= ram_wdata_r;

process(clk_i)

variable set_v    : integer range 0 to CACHE_SETS-1;

variable hit_v    : boolean;

variable hit_way  : integer range 0 to CACHE_WAYS-1;

variable victim_v : integer range 0 to CACHE_WAYS-1;

variable tag_v    : std_logic_vector(11 downto 0);

begin

if rising_edge(clk_i) then

if rst_i = '1' then

state_r     <= IDLE;

data_r      <= (others => (others => (others => '0')));

tag_r       <= (others => (others => (others => '0')));

age_r       <= (others => (others => (others => '0')));

valid_r     <= (others => (others => '0'));

req_addr_r  <= (others => '0');

req_wdata_r <= (others => '0');

req_we_r    <= '0';

write_hit_r <= '0';

victim_r    <= 0;

set_r       <= 0;

tag_req_r   <= (others => '0');

cpu_rdata_r <= (others => '0');

cpu_ready_r <= '0';

hit_r       <= '0';

miss_r      <= '0';

ram_req_r   <= '0';

ram_we_r    <= '0';

ram_addr_r  <= (others => '0');

ram_wdata_r <= (others => '0');

else

cpu_ready_r <= '0';

hit_r       <= '0';

miss_r      <= '0';

case state_r is

when IDLE =>

ram_req_r <= '0';

ram_we_r  <= '0';

if cpu_req_i = '1' and cpu_ready_r = '0' then

set_v := cache_set(cpu_addr_i);

tag_v := cache_tag(cpu_addr_i);

hit_v := false;

hit_way := 0;

victim_v := 0;

for way in 0 to CACHE_WAYS-1 loop

if valid_r(set_v, way) = '1' and tag_r(set_v, way) = tag_v then

hit_v := true;

hit_way := way;

end if;

end loop;

for way in 0 to CACHE_WAYS-1 loop

if valid_r(set_v, way) = '0' then

victim_v := way;

elsif valid_r(set_v, victim_v) = '1' and age_r(set_v, way) > age_r(set_v, victim_v) then

victim_v := way;

end if;

end loop;

req_addr_r  <= cpu_addr_i;

req_wdata_r <= cpu_wdata_i;

req_we_r    <= cpu_we_i;

write_hit_r <= '0';

victim_r    <= victim_v;

set_r       <= set_v;

tag_req_r   <= tag_v;

if hit_v and cpu_we_i = '0' then

cpu_rdata_r <= data_r(set_v, hit_way);

cpu_ready_r <= '1';

hit_r       <= '1';

elsif cpu_we_i = '1' then

if hit_v then

data_r(set_v, hit_way) <= cpu_wdata_i;

victim_v := hit_way;

write_hit_r <= '1';

end if;

ram_req_r   <= '1';

ram_we_r    <= '1';

ram_addr_r  <= cpu_addr_i;

ram_wdata_r <= cpu_wdata_i;

victim_r    <= victim_v;

state_r     <= WAIT_WRITE_GRANT;

if hit_v then

hit_r <= '1';

else

miss_r <= '1';

end if;

else

ram_req_r   <= '1';

ram_we_r    <= '0';

ram_addr_r  <= cpu_addr_i;

ram_wdata_r <= (others => '0');

state_r     <= WAIT_READ_GRANT;

miss_r      <= '1';

end if;

end if;

when WAIT_READ_GRANT =>

ram_req_r <= '1';

ram_we_r  <= '0';

ram_addr_r <= req_addr_r;

if ram_grant_i = '1' then

state_r <= WAIT_READ_DATA;

end if;

when WAIT_READ_DATA =>

ram_req_r <= '0';

ram_we_r  <= '0';

data_r(set_r, victim_r)  <= ram_rdata_i;

tag_r(set_r, victim_r)   <= tag_req_r;

valid_r(set_r, victim_r) <= '1';

age_r(set_r, victim_r)   <= (others => '0');

for way in 0 to CACHE_WAYS-1 loop

if way /= victim_r and valid_r(set_r, way) = '1' then

age_r(set_r, way) <= inc_age(age_r(set_r, way));

end if;

end loop;

cpu_rdata_r <= ram_rdata_i;

cpu_ready_r <= '1';

miss_r      <= '1';

state_r     <= IDLE;

when WAIT_WRITE_GRANT =>

ram_req_r   <= '1';

ram_we_r    <= '1';

ram_addr_r  <= req_addr_r;

ram_wdata_r <= req_wdata_r;

if ram_grant_i = '1' then

if write_hit_r = '0' then

data_r(set_r, victim_r)  <= req_wdata_r;

tag_r(set_r, victim_r)   <= tag_req_r;

valid_r(set_r, victim_r) <= '1';

age_r(set_r, victim_r)   <= (others => '0');

for way in 0 to CACHE_WAYS-1 loop

if way /= victim_r and valid_r(set_r, way) = '1' then

age_r(set_r, way) <= inc_age(age_r(set_r, way));

end if;

end loop;

end if;

cpu_ready_r <= '1';

state_r     <= IDLE;

ram_req_r   <= '0';

ram_we_r    <= '0';

end if;

end case;

end if;

end if;

end process;

end architecture;

cache_4way_structural_doc.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

-- Documentation-oriented structural cache model.

-- It is intentionally split into named blocks so Quartus RTL Viewer can draw

-- a readable one-page cache structure instead of a 75-page primitive netlist.

entity cache_data_way_doc is

port (

clk_i   : in  std_logic;

set_i   : in  std_logic_vector(3 downto 0);

we_i    : in  std_logic;

data_i  : in  word_t;

data_o  : out word_t

);

end entity;

architecture rtl of cache_data_way_doc is

type data_mem_t is array (0 to CACHE_SETS-1) of word_t;

signal mem_r : data_mem_t := (others => (others => '0'));

begin

process(clk_i)

begin

if rising_edge(clk_i) then

if we_i = '1' then

mem_r(to_integer(unsigned(set_i))) <= data_i;

end if;

end if;

end process;

data_o <= mem_r(to_integer(unsigned(set_i)));

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_tag_way_doc is

port (

clk_i   : in  std_logic;

set_i   : in  std_logic_vector(3 downto 0);

we_i    : in  std_logic;

tag_i   : in  std_logic_vector(11 downto 0);

tag_o   : out std_logic_vector(11 downto 0)

);

end entity;

architecture rtl of cache_tag_way_doc is

type tag_mem_t is array (0 to CACHE_SETS-1) of std_logic_vector(11 downto 0);

signal mem_r : tag_mem_t := (others => (others => '0'));

begin

process(clk_i)

begin

if rising_edge(clk_i) then

if we_i = '1' then

mem_r(to_integer(unsigned(set_i))) <= tag_i;

end if;

end if;

end process;

tag_o <= mem_r(to_integer(unsigned(set_i)));

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_valid_way_doc is

port (

clk_i    : in  std_logic;

rst_i    : in  std_logic;

set_i    : in  std_logic_vector(3 downto 0);

we_i     : in  std_logic;

valid_i  : in  std_logic;

valid_o  : out std_logic

);

end entity;

architecture rtl of cache_valid_way_doc is

type valid_mem_t is array (0 to CACHE_SETS-1) of std_logic;

signal mem_r : valid_mem_t := (others => '0');

begin

process(clk_i)

begin

if rising_edge(clk_i) then

if rst_i = '1' then

mem_r <= (others => '0');

elsif we_i = '1' then

mem_r(to_integer(unsigned(set_i))) <= valid_i;

end if;

end if;

end process;

valid_o <= mem_r(to_integer(unsigned(set_i)));

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_age_way_doc is

port (

clk_i   : in  std_logic;

rst_i   : in  std_logic;

set_i   : in  std_logic_vector(3 downto 0);

we_i    : in  std_logic;

inc_i   : in  std_logic;

age_i   : in  unsigned(1 downto 0);

age_o   : out unsigned(1 downto 0)

);

end entity;

architecture rtl of cache_age_way_doc is

type age_mem_t is array (0 to CACHE_SETS-1) of unsigned(1 downto 0);

signal mem_r : age_mem_t := (others => (others => '0'));

begin

process(clk_i)

variable idx_v : integer range 0 to CACHE_SETS-1;

begin

if rising_edge(clk_i) then

idx_v := to_integer(unsigned(set_i));

if rst_i = '1' then

mem_r <= (others => (others => '0'));

elsif we_i = '1' then

mem_r(idx_v) <= age_i;

elsif inc_i = '1' then

mem_r(idx_v) <= sat_inc2(mem_r(idx_v));

end if;

end if;

end process;

age_o <= mem_r(to_integer(unsigned(set_i)));

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_tag_compare_doc is

port (

req_tag_i  : in  std_logic_vector(11 downto 0);

way_tag_i  : in  std_logic_vector(11 downto 0);

equal_o    : out std_logic

);

end entity;

architecture rtl of cache_tag_compare_doc is

begin

equal_o <= '1' when way_tag_i = req_tag_i else '0';

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_hit_miss_logic_doc is

port (

tag_equal0_i : in  std_logic;

tag_equal1_i : in  std_logic;

tag_equal2_i : in  std_logic;

tag_equal3_i : in  std_logic;

valid0_i     : in  std_logic;

valid1_i     : in  std_logic;

valid2_i     : in  std_logic;

valid3_i     : in  std_logic;

way_hit_o    : out std_logic_vector(3 downto 0);

hit_o        : out std_logic;

miss_o       : out std_logic

);

end entity;

architecture rtl of cache_hit_miss_logic_doc is

signal way_hit_s : std_logic_vector(3 downto 0);

signal hit_s     : std_logic;

begin

-- These four ANDs plus OR/NOT are the useful "primitives" to show in the report.

way_hit_s(0) <= tag_equal0_i and valid0_i;

way_hit_s(1) <= tag_equal1_i and valid1_i;

way_hit_s(2) <= tag_equal2_i and valid2_i;

way_hit_s(3) <= tag_equal3_i and valid3_i;

hit_s        <= way_hit_s(0) or way_hit_s(1) or way_hit_s(2) or way_hit_s(3);

way_hit_o <= way_hit_s;

hit_o     <= hit_s;

miss_o    <= not hit_s;

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_hit_mux_doc is

port (

hit0_i   : in  std_logic;

hit1_i   : in  std_logic;

hit2_i   : in  std_logic;

hit3_i   : in  std_logic;

data0_i  : in  word_t;

data1_i  : in  word_t;

data2_i  : in  word_t;

data3_i  : in  word_t;

hit_way_o: out std_logic_vector(1 downto 0);

data_o   : out word_t

);

end entity;

architecture rtl of cache_hit_mux_doc is

begin

process(hit0_i, hit1_i, hit2_i, hit3_i, data0_i, data1_i, data2_i, data3_i)

begin

if hit0_i = '1' then

data_o    <= data0_i;

hit_way_o <= "00";

elsif hit1_i = '1' then

data_o    <= data1_i;

hit_way_o <= "01";

elsif hit2_i = '1' then

data_o    <= data2_i;

hit_way_o <= "10";

else

data_o    <= data3_i;

hit_way_o <= "11";

end if;

end process;

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_address_split_doc is

port (

cpu_addr_i : in  addr_t;

tag_o      : out std_logic_vector(11 downto 0);

set_o      : out std_logic_vector(3 downto 0)

);

end entity;

architecture rtl of cache_address_split_doc is

begin

tag_o <= cpu_addr_i(15 downto 4);

set_o <= cpu_addr_i(3 downto 0);

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_victim_select_doc is

port (

valid0_i  : in  std_logic;

valid1_i  : in  std_logic;

valid2_i  : in  std_logic;

valid3_i  : in  std_logic;

age0_i    : in  unsigned(1 downto 0);

age1_i    : in  unsigned(1 downto 0);

age2_i    : in  unsigned(1 downto 0);

age3_i    : in  unsigned(1 downto 0);

victim_o  : out std_logic_vector(1 downto 0);

onehot_o  : out std_logic_vector(3 downto 0)

);

end entity;

architecture rtl of cache_victim_select_doc is

begin

process(valid0_i, valid1_i, valid2_i, valid3_i, age0_i, age1_i, age2_i, age3_i)

begin

if valid0_i = '0' then

victim_o <= "00";

onehot_o <= "0001";

elsif valid1_i = '0' then

victim_o <= "01";

onehot_o <= "0010";

elsif valid2_i = '0' then

victim_o <= "10";

onehot_o <= "0100";

elsif valid3_i = '0' then

victim_o <= "11";

onehot_o <= "1000";

elsif age1_i >= age0_i and age1_i >= age2_i and age1_i >= age3_i then

victim_o <= "01";

onehot_o <= "0010";

elsif age2_i >= age0_i and age2_i >= age1_i and age2_i >= age3_i then

victim_o <= "10";

onehot_o <= "0100";

elsif age3_i >= age0_i and age3_i >= age1_i and age3_i >= age2_i then

victim_o <= "11";

onehot_o <= "1000";

else

victim_o <= "00";

onehot_o <= "0001";

end if;

end process;

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_control_doc is

port (

clk_i       : in  std_logic;

rst_i       : in  std_logic;

cpu_req_i   : in  std_logic;

cpu_we_i    : in  std_logic;

hit_i       : in  std_logic;

hit_way_i   : in  std_logic_vector(1 downto 0);

victim_i    : in  std_logic_vector(1 downto 0);

victim_oh_i : in  std_logic_vector(3 downto 0);

ram_grant_i : in  std_logic;

way_we_o    : out std_logic_vector(3 downto 0);

age_inc_o   : out std_logic_vector(3 downto 0);

ram_req_o   : out std_logic;

ram_we_o    : out std_logic;

fill_sel_o  : out std_logic;

cpu_ready_o : out std_logic

);

end entity;

architecture rtl of cache_control_doc is

type state_t is (IDLE, HIT_READ, MISS_READ_REQ, MISS_FILL, WRITE_THROUGH);

signal state_r : state_t := IDLE;

begin

process(clk_i)

begin

if rising_edge(clk_i) then

if rst_i = '1' then

state_r <= IDLE;

else

case state_r is

when IDLE =>

if cpu_req_i = '1' and cpu_we_i = '0' and hit_i = '1' then

state_r <= HIT_READ;

elsif cpu_req_i = '1' and cpu_we_i = '0' then

state_r <= MISS_READ_REQ;

elsif cpu_req_i = '1' and cpu_we_i = '1' then

state_r <= WRITE_THROUGH;

else

state_r <= IDLE;

end if;

when HIT_READ =>

state_r <= IDLE;

when MISS_READ_REQ =>

if ram_grant_i = '1' then

state_r <= MISS_FILL;

end if;

when MISS_FILL =>

state_r <= IDLE;

when WRITE_THROUGH =>

if ram_grant_i = '1' then

state_r <= IDLE;

end if;

end case;

end if;

end if;

end process;

ram_req_o   <= '1' when state_r = MISS_READ_REQ or state_r = WRITE_THROUGH else '0';

ram_we_o    <= '1' when state_r = WRITE_THROUGH else '0';

fill_sel_o  <= '1' when state_r = MISS_FILL else '0';

cpu_ready_o <= '1' when state_r = HIT_READ or state_r = MISS_FILL or (state_r = WRITE_THROUGH and ram_grant_i = '1') else '0';

way_we_o <= victim_oh_i when state_r = MISS_FILL else

"0001" when state_r = WRITE_THROUGH and hit_i = '1' and hit_way_i = "00" else

"0010" when state_r = WRITE_THROUGH and hit_i = '1' and hit_way_i = "01" else

"0100" when state_r = WRITE_THROUGH and hit_i = '1' and hit_way_i = "10" else

"1000" when state_r = WRITE_THROUGH and hit_i = '1' and hit_way_i = "11" else

victim_oh_i when state_r = WRITE_THROUGH else

"0000";

age_inc_o <= not victim_oh_i when state_r = MISS_FILL else "0000";

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_ram_interface_doc is

port (

cpu_addr_i   : in  addr_t;

cpu_wdata_i  : in  word_t;

cpu_we_i     : in  std_logic;

ram_req_i    : in  std_logic;

ram_we_i     : in  std_logic;

ram_req_o    : out std_logic;

ram_we_o     : out std_logic;

ram_addr_o   : out addr_t;

ram_wdata_o  : out word_t

);

end entity;

architecture rtl of cache_ram_interface_doc is

begin

ram_req_o   <= ram_req_i;

ram_we_o    <= ram_we_i and cpu_we_i;

ram_addr_o  <= cpu_addr_i;

ram_wdata_o <= cpu_wdata_i;

end architecture;

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cache_4way_structural_doc is

port (

clk_i        : in  std_logic;

rst_i        : in  std_logic;

cpu_req_i    : in  std_logic;

cpu_we_i     : in  std_logic;

cpu_addr_i   : in  addr_t;

cpu_wdata_i  : in  word_t;

cpu_rdata_o  : out word_t;

cpu_ready_o  : out std_logic;

hit_o        : out std_logic;

miss_o       : out std_logic;

ram_req_o    : out std_logic;

ram_we_o     : out std_logic;

ram_addr_o   : out addr_t;

ram_wdata_o  : out word_t;

ram_rdata_i  : in  word_t;

ram_grant_i  : in  std_logic

);

end entity;

architecture structural of cache_4way_structural_doc is

signal set_s       : std_logic_vector(3 downto 0);

signal tag_s       : std_logic_vector(11 downto 0);

signal data_in_s   : word_t;

signal data0_s     : word_t;

signal data1_s     : word_t;

signal data2_s     : word_t;

signal data3_s     : word_t;

signal tag0_s      : std_logic_vector(11 downto 0);

signal tag1_s      : std_logic_vector(11 downto 0);

signal tag2_s      : std_logic_vector(11 downto 0);

signal tag3_s      : std_logic_vector(11 downto 0);

signal valid0_s    : std_logic;

signal valid1_s    : std_logic;

signal valid2_s    : std_logic;

signal valid3_s    : std_logic;

signal age0_s      : unsigned(1 downto 0);

signal age1_s      : unsigned(1 downto 0);

signal age2_s      : unsigned(1 downto 0);

signal age3_s      : unsigned(1 downto 0);

signal tag_equal0_s: std_logic;

signal tag_equal1_s: std_logic;

signal tag_equal2_s: std_logic;

signal tag_equal3_s: std_logic;

signal way_hit_s   : std_logic_vector(3 downto 0);

signal hit_any_s   : std_logic;

signal miss_s      : std_logic;

signal hit_way_s   : std_logic_vector(1 downto 0);

signal victim_s    : std_logic_vector(1 downto 0);

signal victim_oh_s : std_logic_vector(3 downto 0);

signal way_we_s    : std_logic_vector(3 downto 0);

signal age_inc_s   : std_logic_vector(3 downto 0);

signal fill_sel_s  : std_logic;

signal ram_req_s   : std_logic;

signal ram_we_s    : std_logic;

signal cache_data_s: word_t;

begin

data_in_s <= ram_rdata_i when fill_sel_s = '1' else cpu_wdata_i;

hit_o     <= hit_any_s;

miss_o    <= miss_s;

cpu_rdata_o <= cache_data_s when hit_any_s = '1' else ram_rdata_i;

U_ADDRESS_SPLIT_TAG_SET : entity work.cache_address_split_doc

port map (

cpu_addr_i => cpu_addr_i,

tag_o      => tag_s,

set_o      => set_s

);

U_CACHE_CONTROL : entity work.cache_control_doc

port map (

clk_i       => clk_i,

rst_i       => rst_i,

cpu_req_i   => cpu_req_i,

cpu_we_i    => cpu_we_i,

hit_i       => hit_any_s,

hit_way_i   => hit_way_s,

victim_i    => victim_s,

victim_oh_i => victim_oh_s,

ram_grant_i => ram_grant_i,

way_we_o    => way_we_s,

age_inc_o   => age_inc_s,

ram_req_o   => ram_req_s,

ram_we_o    => ram_we_s,

fill_sel_o  => fill_sel_s,

cpu_ready_o => cpu_ready_o

);

U_VICTIM_SELECT : entity work.cache_victim_select_doc

port map (

valid0_i => valid0_s,

valid1_i => valid1_s,

valid2_i => valid2_s,

valid3_i => valid3_s,

age0_i   => age0_s,

age1_i   => age1_s,

age2_i   => age2_s,

age3_i   => age3_s,

victim_o => victim_s,

onehot_o => victim_oh_s

);

U_RAM_INTERFACE : entity work.cache_ram_interface_doc

port map (

cpu_addr_i  => cpu_addr_i,

cpu_wdata_i => cpu_wdata_i,

cpu_we_i    => cpu_we_i,

ram_req_i   => ram_req_s,

ram_we_i    => ram_we_s,

ram_req_o   => ram_req_o,

ram_we_o    => ram_we_o,

ram_addr_o  => ram_addr_o,

ram_wdata_o => ram_wdata_o

);

U_DATA_MUX : entity work.cache_hit_mux_doc

port map (

hit0_i    => way_hit_s(0),

hit1_i    => way_hit_s(1),

hit2_i    => way_hit_s(2),

hit3_i    => way_hit_s(3),

data0_i   => data0_s,

data1_i   => data1_s,

data2_i   => data2_s,

data3_i   => data3_s,

hit_way_o => hit_way_s,

data_o    => cache_data_s

);

U_DATA_WAY0 : entity work.cache_data_way_doc port map (clk_i => clk_i, set_i => set_s, we_i => way_we_s(0), data_i => data_in_s, data_o => data0_s);

U_DATA_WAY1 : entity work.cache_data_way_doc port map (clk_i => clk_i, set_i => set_s, we_i => way_we_s(1), data_i => data_in_s, data_o => data1_s);

U_DATA_WAY2 : entity work.cache_data_way_doc port map (clk_i => clk_i, set_i => set_s, we_i => way_we_s(2), data_i => data_in_s, data_o => data2_s);

U_DATA_WAY3 : entity work.cache_data_way_doc port map (clk_i => clk_i, set_i => set_s, we_i => way_we_s(3), data_i => data_in_s, data_o => data3_s);

U_TAG_WAY0 : entity work.cache_tag_way_doc port map (clk_i => clk_i, set_i => set_s, we_i => way_we_s(0), tag_i => tag_s, tag_o => tag0_s);

U_TAG_WAY1 : entity work.cache_tag_way_doc port map (clk_i => clk_i, set_i => set_s, we_i => way_we_s(1), tag_i => tag_s, tag_o => tag1_s);

U_TAG_WAY2 : entity work.cache_tag_way_doc port map (clk_i => clk_i, set_i => set_s, we_i => way_we_s(2), tag_i => tag_s, tag_o => tag2_s);

U_TAG_WAY3 : entity work.cache_tag_way_doc port map (clk_i => clk_i, set_i => set_s, we_i => way_we_s(3), tag_i => tag_s, tag_o => tag3_s);

U_VALID_WAY0 : entity work.cache_valid_way_doc port map (clk_i => clk_i, rst_i => rst_i, set_i => set_s, we_i => way_we_s(0), valid_i => '1', valid_o => valid0_s);

U_VALID_WAY1 : entity work.cache_valid_way_doc port map (clk_i => clk_i, rst_i => rst_i, set_i => set_s, we_i => way_we_s(1), valid_i => '1', valid_o => valid1_s);

U_VALID_WAY2 : entity work.cache_valid_way_doc port map (clk_i => clk_i, rst_i => rst_i, set_i => set_s, we_i => way_we_s(2), valid_i => '1', valid_o => valid2_s);

U_VALID_WAY3 : entity work.cache_valid_way_doc port map (clk_i => clk_i, rst_i => rst_i, set_i => set_s, we_i => way_we_s(3), valid_i => '1', valid_o => valid3_s);

U_AGE_WAY0 : entity work.cache_age_way_doc port map (clk_i => clk_i, rst_i => rst_i, set_i => set_s, we_i => way_we_s(0), inc_i => age_inc_s(0), age_i => "00", age_o => age0_s);

U_AGE_WAY1 : entity work.cache_age_way_doc port map (clk_i => clk_i, rst_i => rst_i, set_i => set_s, we_i => way_we_s(1), inc_i => age_inc_s(1), age_i => "00", age_o => age1_s);

U_AGE_WAY2 : entity work.cache_age_way_doc port map (clk_i => clk_i, rst_i => rst_i, set_i => set_s, we_i => way_we_s(2), inc_i => age_inc_s(2), age_i => "00", age_o => age2_s);

U_AGE_WAY3 : entity work.cache_age_way_doc port map (clk_i => clk_i, rst_i => rst_i, set_i => set_s, we_i => way_we_s(3), inc_i => age_inc_s(3), age_i => "00", age_o => age3_s);

U_TAG_COMPARE0 : entity work.cache_tag_compare_doc port map (req_tag_i => tag_s, way_tag_i => tag0_s, equal_o => tag_equal0_s);

U_TAG_COMPARE1 : entity work.cache_tag_compare_doc port map (req_tag_i => tag_s, way_tag_i => tag1_s, equal_o => tag_equal1_s);

U_TAG_COMPARE2 : entity work.cache_tag_compare_doc port map (req_tag_i => tag_s, way_tag_i => tag2_s, equal_o => tag_equal2_s);

U_TAG_COMPARE3 : entity work.cache_tag_compare_doc port map (req_tag_i => tag_s, way_tag_i => tag3_s, equal_o => tag_equal3_s);

U_HIT_MISS_LOGIC_AND_OR_NOT : entity work.cache_hit_miss_logic_doc

port map (

tag_equal0_i => tag_equal0_s,

tag_equal1_i => tag_equal1_s,

tag_equal2_i => tag_equal2_s,

tag_equal3_i => tag_equal3_s,

valid0_i     => valid0_s,

valid1_i     => valid1_s,

valid2_i     => valid2_s,

valid3_i     => valid3_s,

way_hit_o    => way_hit_s,

hit_o        => hit_any_s,

miss_o       => miss_s

);

end architecture;

cpu_core.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity cpu_core is

port (

clk_i          : in  std_logic;

rst_i          : in  std_logic;

rom_addr_o     : out addr_t;

rom_en_o       : out std_logic;

rom_data_i     : in  word_t;

cache_req_o    : out std_logic;

cache_we_o     : out std_logic;

cache_addr_o   : out addr_t;

cache_wdata_o  : out word_t;

cache_rdata_i  : in  word_t;

cache_ready_i  : in  std_logic;

cache_hit_i    : in  std_logic;

cache_miss_i   : in  std_logic;

bp_query_o     : out std_logic;

bp_pc_query_o  : out addr_t;

bp_hist_i      : in  std_logic_vector(1 downto 0);

bp_pred_i      : in  std_logic;

bp_target_i    : in  addr_t;

bp_update_o    : out std_logic;

bp_pc_update_o : out addr_t;

bp_hist_o      : out std_logic_vector(1 downto 0);

bp_taken_o     : out std_logic;

bp_target_o    : out addr_t;

halt_o         : out std_logic;

dbg_state_o    : out std_logic_vector(4 downto 0);

dbg_pc_o       : out addr_t;

dbg_ir0_o      : out word_t;

dbg_ir1_o      : out word_t;

dbg_r1_o       : out word_t;

dbg_r2_o       : out word_t;

dbg_r3_o       : out word_t;

dbg_r4_o       : out word_t;

dbg_r5_o       : out word_t;

dbg_r6_o       : out word_t;

dbg_r7_o       : out word_t;

dbg_flags_o    : out std_logic_vector(3 downto 0);

dbg_sp_o       : out unsigned(2 downto 0)

);

end entity;

architecture rtl of cpu_core is

subtype state_code_t is std_logic_vector(4 downto 0);

type state_t is (

S_RESET,

S_FETCH0_REQ,

S_FETCH0_WAIT,

S_FETCH0_LATCH,

S_FETCH1_REQ,

S_FETCH1_WAIT,

S_FETCH1_LATCH,

S_DECODE,

S_CACHE_READ,

S_CACHE_WRITE,

S_ALU_SETUP_REG,

S_ALU_WRITE,

S_RF_WRITE_MEM,

S_STACK_PUSH,

S_STACK_POP,

S_RF_WRITE_POP,

S_BRANCH_JMP,

S_BRANCH_JZ,

S_FINISH,

S_HALT

);

signal state_r       : state_t := S_RESET;

signal pc_r          : addr_t := (others => '0');

signal ir0_r         : word_t := (others => '0');

signal ir1_r         : word_t := (others => '0');

signal cur_reg_r     : reg_idx_t := (others => '0');

signal cur_opcode_r  : std_logic_vector(7 downto 0) := OP_HLT;

signal rom_addr_r    : addr_t := (others => '0');

signal rom_en_r      : std_logic := '0';

signal rf_we         : std_logic;

signal rf_wr_addr    : reg_idx_t;

signal rf_rd_a       : reg_idx_t;

signal rf_rd_b       : reg_idx_t;

signal rf_din        : word_t;

signal rf_a          : word_t;

signal rf_b          : word_t;

signal alu_a_r       : word_t := (others => '0');

signal alu_b_r       : word_t := (others => '0');

signal alu_op_r      : std_logic_vector(2 downto 0) := ALU_PASS_A;

signal alu_y         : word_t;

signal alu_z         : std_logic;

signal alu_s         : std_logic;

signal alu_c         : std_logic;

signal alu_o         : std_logic;

signal flag_we       : std_logic;

signal flag_z        : std_logic;

signal flag_s        : std_logic;

signal flag_c        : std_logic;

signal flag_o        : std_logic;

signal stack_push    : std_logic;

signal stack_pop     : std_logic;

signal stack_dout    : word_t;

signal stack_sp      : unsigned(2 downto 0);

signal stack_empty   : std_logic;

signal stack_full    : std_logic;

signal r1_dbg        : word_t;

signal r2_dbg        : word_t;

signal r3_dbg        : word_t;

signal r4_dbg        : word_t;

signal r5_dbg        : word_t;

signal r6_dbg        : word_t;

signal r7_dbg        : word_t;

function state_code(s : state_t) return state_code_t is

begin

case s is

when S_RESET        => return "00000";

when S_FETCH0_REQ   => return "00001";

when S_FETCH0_WAIT  => return "00010";

when S_FETCH0_LATCH => return "00011";

when S_FETCH1_REQ   => return "00100";

when S_FETCH1_WAIT  => return "00101";

when S_FETCH1_LATCH => return "00110";

when S_DECODE       => return "00111";

when S_CACHE_READ   => return "01000";

when S_CACHE_WRITE  => return "01001";

when S_ALU_SETUP_REG=> return "01010";

when S_ALU_WRITE    => return "01011";

when S_RF_WRITE_MEM => return "01100";

when S_STACK_PUSH   => return "01101";

when S_STACK_POP    => return "01110";

when S_RF_WRITE_POP => return "01111";

when S_BRANCH_JMP   => return "10000";

when S_BRANCH_JZ    => return "10001";

when S_FINISH       => return "10010";

when S_HALT         => return "11111";

end case;

end function;

begin

rom_addr_o <= rom_addr_r;

rom_en_o   <= rom_en_r;

rf_wr_addr <= cur_reg_r;

rf_rd_a    <= cur_reg_r;

rf_rd_b    <= (others => '0');

rf_we      <= '1' when state_r = S_ALU_WRITE or state_r = S_RF_WRITE_MEM or state_r = S_RF_WRITE_POP else '0';

rf_din     <= alu_y when state_r = S_ALU_WRITE else

cache_rdata_i when state_r = S_RF_WRITE_MEM else

stack_dout;

flag_we <= '1' when state_r = S_ALU_WRITE else '0';

stack_push <= '1' when state_r = S_STACK_PUSH else '0';

stack_pop  <= '1' when state_r = S_STACK_POP else '0';

cache_req_o   <= '1' when state_r = S_CACHE_READ or state_r = S_CACHE_WRITE else '0';

cache_we_o    <= '1' when state_r = S_CACHE_WRITE else '0';

cache_addr_o  <= ir1_r;

cache_wdata_o <= rf_a;

bp_query_o     <= '1' when state_r = S_FETCH0_REQ else '0';

bp_pc_query_o  <= pc_r;

bp_update_o    <= '1' when state_r = S_BRANCH_JZ else '0';

bp_pc_update_o <= pc_r;

bp_hist_o      <= bp_hist_i;

bp_taken_o     <= flag_z;

bp_target_o    <= ir1_r;

halt_o      <= '1' when state_r = S_HALT else '0';

dbg_state_o <= state_code(state_r);

dbg_pc_o    <= pc_r;

dbg_ir0_o   <= ir0_r;

dbg_ir1_o   <= ir1_r;

dbg_r1_o    <= r1_dbg;

dbg_r2_o    <= r2_dbg;

dbg_r3_o    <= r3_dbg;

dbg_r4_o    <= r4_dbg;

dbg_r5_o    <= r5_dbg;

dbg_r6_o    <= r6_dbg;

dbg_r7_o    <= r7_dbg;

dbg_flags_o <= flag_z & flag_s & flag_c & flag_o;

dbg_sp_o    <= stack_sp;

U_RF : entity work.reg_file12x16_dbg

port map (

clk_i       => clk_i,

rst_i       => rst_i,

we_i        => rf_we,

wr_addr_i   => rf_wr_addr,

rd_addr_a_i => rf_rd_a,

rd_addr_b_i => rf_rd_b,

din_i       => rf_din,

dout_a_o    => rf_a,

dout_b_o    => rf_b,

r0_o        => open,

r1_o        => r1_dbg,

r2_o        => r2_dbg,

r3_o        => r3_dbg,

r4_o        => r4_dbg,

r5_o        => r5_dbg,

r6_o        => r6_dbg,

r7_o        => r7_dbg,

r8_o        => open,

r9_o        => open,

r10_o       => open,

r11_o       => open

);

U_ALU : entity work.alu_core

port map (

a_i      => alu_a_r,

b_i      => alu_b_r,

flag_s_i => flag_s,

op_i     => alu_op_r,

y_o      => alu_y,

z_o      => alu_z,

s_o      => alu_s,

c_o      => alu_c,

o_o      => alu_o

);

U_FLAGS : entity work.flags_reg

port map (

clk_i => clk_i,

rst_i => rst_i,

we_i  => flag_we,

z_i   => alu_z,

s_i   => alu_s,

c_i   => alu_c,

o_i   => alu_o,

z_o   => flag_z,

s_o   => flag_s,

c_o   => flag_c,

o_o   => flag_o

);

U_STACK : entity work.stack7x16

port map (

clk_i   => clk_i,

rst_i   => rst_i,

push_i  => stack_push,

pop_i   => stack_pop,

din_i   => rf_a,

dout_o  => stack_dout,

sp_o    => stack_sp,

empty_o => stack_empty,

full_o  => stack_full

);

process(clk_i)

variable opcode_v : std_logic_vector(7 downto 0);

variable next_pc  : unsigned(15 downto 0);

begin

if rising_edge(clk_i) then

if rst_i = '1' then

state_r      <= S_RESET;

pc_r         <= (others => '0');

ir0_r        <= (others => '0');

ir1_r        <= (others => '0');

cur_reg_r    <= (others => '0');

cur_opcode_r <= OP_HLT;

rom_addr_r   <= (others => '0');

rom_en_r     <= '0';

alu_a_r      <= (others => '0');

alu_b_r      <= (others => '0');

alu_op_r     <= ALU_PASS_A;

else

rom_en_r <= '1';

next_pc := unsigned(pc_r) + 2;

case state_r is

when S_RESET =>

pc_r       <= (others => '0');

rom_addr_r <= (others => '0');

state_r    <= S_FETCH0_REQ;

when S_FETCH0_REQ =>

rom_addr_r <= pc_r;

state_r    <= S_FETCH0_WAIT;

when S_FETCH0_WAIT =>

state_r <= S_FETCH0_LATCH;

when S_FETCH0_LATCH =>

ir0_r      <= rom_data_i;

rom_addr_r <= std_logic_vector(unsigned(pc_r) + 1);

state_r    <= S_FETCH1_REQ;

when S_FETCH1_REQ =>

state_r <= S_FETCH1_WAIT;

when S_FETCH1_WAIT =>

state_r <= S_FETCH1_LATCH;

when S_FETCH1_LATCH =>

ir1_r   <= rom_data_i;

state_r <= S_DECODE;

when S_DECODE =>

opcode_v     := ir0_r(15 downto 8);

cur_opcode_r <= opcode_v;

cur_reg_r    <= unsigned(ir0_r(7 downto 4));

if opcode_v = OP_MOV_MR or opcode_v = OP_OR or opcode_v = OP_NOR then

state_r <= S_CACHE_READ;

elsif opcode_v = OP_MOV_RM then

state_r <= S_CACHE_WRITE;

elsif opcode_v = OP_SRA or opcode_v = OP_INCS then

state_r <= S_ALU_SETUP_REG;

elsif opcode_v = OP_PUSH then

state_r <= S_STACK_PUSH;

elsif opcode_v = OP_POP then

state_r <= S_STACK_POP;

elsif opcode_v = OP_JMP then

state_r <= S_BRANCH_JMP;

elsif opcode_v = OP_JZ then

state_r <= S_BRANCH_JZ;

elsif opcode_v = OP_HLT then

state_r <= S_HALT;

else

state_r <= S_HALT;

end if;

when S_CACHE_READ =>

if cache_ready_i = '1' then

if cur_opcode_r = OP_MOV_MR then

state_r <= S_RF_WRITE_MEM;

else

alu_a_r <= rf_a;

alu_b_r <= cache_rdata_i;

if cur_opcode_r = OP_OR then

alu_op_r <= ALU_OR;

else

alu_op_r <= ALU_NOR;

end if;

state_r <= S_ALU_WRITE;

end if;

end if;

when S_CACHE_WRITE =>

if cache_ready_i = '1' then

state_r <= S_FINISH;

end if;

when S_ALU_SETUP_REG =>

alu_a_r <= rf_a;

alu_b_r <= (others => '0');

if cur_opcode_r = OP_SRA then

alu_op_r <= ALU_SRA;

else

alu_op_r <= ALU_INCS;

end if;

state_r <= S_ALU_WRITE;

when S_ALU_WRITE =>

state_r <= S_FINISH;

when S_RF_WRITE_MEM =>

state_r <= S_FINISH;

when S_STACK_PUSH =>

state_r <= S_FINISH;

when S_STACK_POP =>

state_r <= S_RF_WRITE_POP;

when S_RF_WRITE_POP =>

state_r <= S_FINISH;

when S_BRANCH_JMP =>

pc_r    <= ir1_r;

state_r <= S_FETCH0_REQ;

when S_BRANCH_JZ =>

if flag_z = '1' then

pc_r <= ir1_r;

else

pc_r <= std_logic_vector(next_pc);

end if;

state_r <= S_FETCH0_REQ;

when S_FINISH =>

pc_r    <= std_logic_vector(next_pc);

state_r <= S_FETCH0_REQ;

when S_HALT =>

state_r <= S_HALT;

end case;

end if;

end if;

end process;

end architecture;

data_image_pkg.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

package data_image_pkg is

constant RAM_INIT : ram256_t := (

16#0A# => x"0000",

16#0B# => x"0000",

16#0C# => x"0000",

16#20# => x"8001",

16#21# => x"00F0",

16#22# => x"0F0F",

16#23# => x"0000",

16#24# => x"1234",

16#25# => x"FFFF",

16#30# => x"0000",

others  => (others => '0')

);

end package;

dma_controller_3word.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity dma_controller_3word is

generic (

BASE_ADDR_G : addr_t := x"000A"

);

port (

clk_i       : in  std_logic;

rst_i       : in  std_logic;

start_i     : in  std_logic;

grant_i     : in  std_logic;

dev_valid_i : in  std_logic;

dev_data_i  : in  word_t;

req_o       : out std_logic;

busy_o      : out std_logic;

done_o      : out std_logic;

ram_we_o    : out std_logic;

ram_addr_o  : out addr_t;

ram_data_o  : out word_t

);

end entity;

architecture rtl of dma_controller_3word is

type state_t is (IDLE, REQ_BUS, WRITE_WORD, FINISH);

signal state_r  : state_t := IDLE;

signal index_r  : unsigned(1 downto 0) := (others => '0');

signal done_r   : std_logic := '0';

begin

busy_o     <= '0' when state_r = IDLE else '1';

req_o      <= '1' when state_r = REQ_BUS or state_r = WRITE_WORD else '0';

ram_we_o   <= '1' when state_r = WRITE_WORD and grant_i = '1' and dev_valid_i = '1' else '0';

ram_addr_o <= std_logic_vector(unsigned(BASE_ADDR_G) + resize(index_r, ADDR_W));

ram_data_o <= dev_data_i;

done_o     <= done_r;

process(clk_i)

begin

if rising_edge(clk_i) then

if rst_i = '1' then

state_r <= IDLE;

index_r <= (others => '0');

done_r  <= '0';

else

case state_r is

when IDLE =>

if start_i = '1' then

index_r <= (others => '0');

done_r  <= '0';

state_r <= REQ_BUS;

end if;

when REQ_BUS =>

if grant_i = '1' then

state_r <= WRITE_WORD;

end if;

when WRITE_WORD =>

if grant_i = '1' and dev_valid_i = '1' then

if index_r = "10" then

state_r <= FINISH;

else

index_r <= index_r + 1;

end if;

end if;

when FINISH =>

done_r  <= '1';

state_r <= IDLE;

end case;

end if;

end if;

end process;

end architecture;

flags_reg.vhd

library ieee;

use ieee.std_logic_1164.all;

entity flags_reg is

port (

clk_i : in  std_logic;

rst_i : in  std_logic;

we_i  : in  std_logic;

z_i   : in  std_logic;

s_i   : in  std_logic;

c_i   : in  std_logic;

o_i   : in  std_logic;

z_o   : out std_logic;

s_o   : out std_logic;

c_o   : out std_logic;

o_o   : out std_logic

);

end entity;

architecture rtl of flags_reg is

signal z_r, s_r, c_r, o_r : std_logic := '0';

begin

z_o <= z_r;

s_o <= s_r;

c_o <= c_r;

o_o <= o_r;

process(clk_i)

begin

if rising_edge(clk_i) then

if rst_i = '1' then

z_r <= '0';

s_r <= '0';

c_r <= '0';

o_r <= '0';

elsif we_i = '1' then

z_r <= z_i;

s_r <= s_i;

c_r <= c_i;

o_r <= o_i;

end if;

end if;

end process;

end architecture;

microcomputer_debug_top.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity microcomputer_debug_top is

port (

CLK              : in  std_logic;

RESET            : in  std_logic;

DMA_START        : in  std_logic;

DMA_VALID        : in  std_logic;

DMA_DATA         : in  word_t;

HALT             : out std_logic;

DMA_DONE         : out std_logic;

DBG_STATE        : out std_logic_vector(4 downto 0);

DBG_PC           : out addr_t;

DBG_IR0          : out word_t;

DBG_IR1          : out word_t;

DBG_R1           : out word_t;

DBG_R2           : out word_t;

DBG_R3           : out word_t;

DBG_R4           : out word_t;

DBG_R5           : out word_t;

DBG_R6           : out word_t;

DBG_R7           : out word_t;

DBG_FLAGS        : out std_logic_vector(3 downto 0);

DBG_SP           : out unsigned(2 downto 0);

DBG_CACHE_HIT    : out std_logic;

DBG_CACHE_MISS   : out std_logic;

DBG_REQ_CPU      : out std_logic;

DBG_REQ_DMA      : out std_logic;

DBG_GNT_CPU      : out std_logic;

DBG_GNT_DMA      : out std_logic;

DBG_RAM_WE       : out std_logic;

DBG_RAM_ADDR     : out addr_t;

DBG_RAM_WDATA    : out word_t;

DBG_RAM_RDATA    : out word_t;

DBG_BP_HIST      : out std_logic_vector(1 downto 0);

DBG_BP_PRED      : out std_logic;

DBG_BP_TARGET    : out addr_t

);

end entity;

architecture structural of microcomputer_debug_top is

begin

U_SYSTEM : entity work.system_core

port map (

clk_i            => CLK,

rst_i            => RESET,

dma_start_i      => DMA_START,

dma_valid_i      => DMA_VALID,

dma_data_i       => DMA_DATA,

halt_o           => HALT,

dma_done_o       => DMA_DONE,

dbg_state_o      => DBG_STATE,

dbg_pc_o         => DBG_PC,

dbg_ir0_o        => DBG_IR0,

dbg_ir1_o        => DBG_IR1,

dbg_r1_o         => DBG_R1,

dbg_r2_o         => DBG_R2,

dbg_r3_o         => DBG_R3,

dbg_r4_o         => DBG_R4,

dbg_r5_o         => DBG_R5,

dbg_r6_o         => DBG_R6,

dbg_r7_o         => DBG_R7,

dbg_flags_o      => DBG_FLAGS,

dbg_sp_o         => DBG_SP,

dbg_cache_hit_o  => DBG_CACHE_HIT,

dbg_cache_miss_o => DBG_CACHE_MISS,

dbg_req_cpu_o    => DBG_REQ_CPU,

dbg_req_dma_o    => DBG_REQ_DMA,

dbg_gnt_cpu_o    => DBG_GNT_CPU,

dbg_gnt_dma_o    => DBG_GNT_DMA,

dbg_ram_we_o     => DBG_RAM_WE,

dbg_ram_addr_o   => DBG_RAM_ADDR,

dbg_ram_wdata_o  => DBG_RAM_WDATA,

dbg_ram_rdata_o  => DBG_RAM_RDATA,

dbg_bp_hist_o    => DBG_BP_HIST,

dbg_bp_pred_o    => DBG_BP_PRED,

dbg_bp_target_o  => DBG_BP_TARGET

);

end architecture;

microcomputer_pkg.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

package microcomputer_pkg is

constant DATA_W      : positive := 16;

constant ADDR_W      : positive := 16;

constant REG_COUNT   : positive := 12;

constant STACK_DEPTH : positive := 7;

constant CACHE_SETS  : positive := 16;

constant CACHE_WAYS  : positive := 4;

subtype word_t    is std_logic_vector(DATA_W-1 downto 0);

subtype addr_t    is std_logic_vector(ADDR_W-1 downto 0);

subtype reg_idx_t is unsigned(3 downto 0);

type ram256_t is array (0 to 255) of word_t;

constant ALU_PASS_A : std_logic_vector(2 downto 0) := "000";

constant ALU_OR     : std_logic_vector(2 downto 0) := "001";

constant ALU_NOR    : std_logic_vector(2 downto 0) := "010";

constant ALU_SRA    : std_logic_vector(2 downto 0) := "011";

constant ALU_INCS   : std_logic_vector(2 downto 0) := "100";

constant OP_HLT     : std_logic_vector(7 downto 0) := x"00";

constant OP_MOV_MR  : std_logic_vector(7 downto 0) := x"01";

constant OP_MOV_RM  : std_logic_vector(7 downto 0) := x"02";

constant OP_OR      : std_logic_vector(7 downto 0) := x"03";

constant OP_NOR     : std_logic_vector(7 downto 0) := x"04";

constant OP_SRA     : std_logic_vector(7 downto 0) := x"05";

constant OP_INCS    : std_logic_vector(7 downto 0) := x"06";

constant OP_PUSH    : std_logic_vector(7 downto 0) := x"07";

constant OP_POP     : std_logic_vector(7 downto 0) := x"08";

constant OP_JMP     : std_logic_vector(7 downto 0) := x"09";

constant OP_JZ      : std_logic_vector(7 downto 0) := x"0A";

function sat_inc2(v : unsigned(1 downto 0)) return unsigned;

end package;

package body microcomputer_pkg is

function sat_inc2(v : unsigned(1 downto 0)) return unsigned is

begin

if v = "11" then

return v;

else

return v + 1;

end if;

end function;

end package body;

microcomputer_top.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity microcomputer_top is

port (

CLK              : in  std_logic;

RESET            : in  std_logic;

DMA_START        : in  std_logic;

DMA_VALID        : in  std_logic;

DMA_DATA         : in  word_t;

HALT             : out std_logic;

DMA_DONE         : out std_logic

);

end entity;

architecture structural of microcomputer_top is

begin

U_SYSTEM : entity work.system_core

port map (

clk_i            => CLK,

rst_i            => RESET,

dma_start_i      => DMA_START,

dma_valid_i      => DMA_VALID,

dma_data_i       => DMA_DATA,

halt_o           => HALT,

dma_done_o       => DMA_DONE,

dbg_state_o      => open,

dbg_pc_o         => open,

dbg_ir0_o        => open,

dbg_ir1_o        => open,

dbg_r1_o         => open,

dbg_r2_o         => open,

dbg_r3_o         => open,

dbg_r4_o         => open,

dbg_r5_o         => open,

dbg_r6_o         => open,

dbg_r7_o         => open,

dbg_flags_o      => open,

dbg_sp_o         => open,

dbg_cache_hit_o  => open,

dbg_cache_miss_o => open,

dbg_req_cpu_o    => open,

dbg_req_dma_o    => open,

dbg_gnt_cpu_o    => open,

dbg_gnt_dma_o    => open,

dbg_ram_we_o     => open,

dbg_ram_addr_o   => open,

dbg_ram_wdata_o  => open,

dbg_ram_rdata_o  => open,

dbg_bp_hist_o    => open,

dbg_bp_pred_o    => open,

dbg_bp_target_o  => open

);

end architecture;

program_image_pkg.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

package program_image_pkg is

constant ROM_INIT : ram256_t := (

16#00# => x"0110", -- MOV 0020h, R1

16#01# => x"0020",

16#02# => x"0510", -- SRA R1

16#03# => x"0000",

16#04# => x"0610", -- INCS R1

16#05# => x"0000",

16#06# => x"0710", -- PUSH R1

16#07# => x"0000",

16#08# => x"0120", -- MOV 0021h, R2

16#09# => x"0021",

16#0A# => x"0320", -- OR R2, 0022h

16#0B# => x"0022",

16#0C# => x"0420", -- NOR R2, 0023h

16#0D# => x"0023",

16#0E# => x"0830", -- POP R3

16#0F# => x"0000",

16#10# => x"0230", -- MOV R3, 0030h

16#11# => x"0030",

16#12# => x"0140", -- MOV 0025h, R4

16#13# => x"0025",

16#14# => x"0440", -- NOR R4, 0025h

16#15# => x"0025",

16#16# => x"0A00", -- JZ 001Ah

16#17# => x"001A",

16#18# => x"0150", -- MOV 0024h, R5 (must be skipped)

16#19# => x"0024",

16#1A# => x"0900", -- JMP 001Eh

16#1B# => x"001E",

16#1C# => x"0160", -- MOV 0024h, R6 (must be skipped)

16#1D# => x"0024",

16#1E# => x"0170", -- MOV 000Ah, R7

16#1F# => x"000A",

16#20# => x"0000", -- HLT

16#21# => x"0000",

others  => (others => '0')

);

end package;

ram_sync.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

use work.data_image_pkg.all;

entity ram_sync is

port (

clk_i  : in  std_logic;

en_i   : in  std_logic;

we_i   : in  std_logic;

addr_i : in  addr_t;

din_i  : in  word_t;

dout_o : out word_t

);

end entity;

architecture rtl of ram_sync is

signal mem : ram256_t := RAM_INIT;

signal q_r : word_t := (others => '0');

begin

dout_o <= q_r;

process(clk_i)

variable idx : integer;

begin

if rising_edge(clk_i) then

if en_i = '1' then

idx := to_integer(unsigned(addr_i(7 downto 0)));

if we_i = '1' then

mem(idx) <= din_i;

end if;

q_r <= mem(idx);

end if;

end if;

end process;

end architecture;

reg_file12x16.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity reg_file12x16 is

port (

clk_i      : in  std_logic;

rst_i      : in  std_logic;

we_i       : in  std_logic;

wr_addr_i  : in  reg_idx_t;

rd_addr_a_i: in  reg_idx_t;

rd_addr_b_i: in  reg_idx_t;

din_i      : in  word_t;

dout_a_o   : out word_t;

dout_b_o   : out word_t

);

end entity;

architecture rtl of reg_file12x16 is

type reg_array_t is array (0 to REG_COUNT-1) of word_t;

signal regs : reg_array_t := (others => (others => '0'));

function read_reg(mem : reg_array_t; idx : reg_idx_t) return word_t is

begin

if to_integer(idx) < REG_COUNT then

return mem(to_integer(idx));

else

return (others => '0');

end if;

end function;

begin

dout_a_o <= read_reg(regs, rd_addr_a_i);

dout_b_o <= read_reg(regs, rd_addr_b_i);

process(clk_i)

begin

if rising_edge(clk_i) then

if rst_i = '1' then

regs <= (others => (others => '0'));

elsif we_i = '1' and to_integer(wr_addr_i) < REG_COUNT then

regs(to_integer(wr_addr_i)) <= din_i;

end if;

end if;

end process;

end architecture;

reg_file12x16_dbg.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity reg_file12x16_dbg is

port (

clk_i       : in  std_logic;

rst_i       : in  std_logic;

we_i        : in  std_logic;

wr_addr_i   : in  reg_idx_t;

rd_addr_a_i : in  reg_idx_t;

rd_addr_b_i : in  reg_idx_t;

din_i       : in  word_t;

dout_a_o    : out word_t;

dout_b_o    : out word_t;

r0_o        : out word_t;

r1_o        : out word_t;

r2_o        : out word_t;

r3_o        : out word_t;

r4_o        : out word_t;

r5_o        : out word_t;

r6_o        : out word_t;

r7_o        : out word_t;

r8_o        : out word_t;

r9_o        : out word_t;

r10_o       : out word_t;

r11_o       : out word_t

);

end entity;

architecture rtl of reg_file12x16_dbg is

type reg_array_t is array (0 to REG_COUNT-1) of word_t;

signal regs : reg_array_t := (others => (others => '0'));

function read_reg(mem : reg_array_t; idx : reg_idx_t) return word_t is

begin

if to_integer(idx) < REG_COUNT then

return mem(to_integer(idx));

else

return (others => '0');

end if;

end function;

begin

dout_a_o <= read_reg(regs, rd_addr_a_i);

dout_b_o <= read_reg(regs, rd_addr_b_i);

r0_o  <= regs(0);

r1_o  <= regs(1);

r2_o  <= regs(2);

r3_o  <= regs(3);

r4_o  <= regs(4);

r5_o  <= regs(5);

r6_o  <= regs(6);

r7_o  <= regs(7);

r8_o  <= regs(8);

r9_o  <= regs(9);

r10_o <= regs(10);

r11_o <= regs(11);

process(clk_i)

begin

if rising_edge(clk_i) then

if rst_i = '1' then

regs <= (others => (others => '0'));

elsif we_i = '1' and to_integer(wr_addr_i) < REG_COUNT then

regs(to_integer(wr_addr_i)) <= din_i;

end if;

end if;

end process;

end architecture;

rom_sync.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

use work.program_image_pkg.all;

entity rom_sync is

port (

clk_i  : in  std_logic;

en_i   : in  std_logic;

addr_i : in  addr_t;

data_o : out word_t

);

end entity;

architecture rtl of rom_sync is

signal q_r : word_t := (others => '0');

begin

data_o <= q_r;

process(clk_i)

variable idx : integer;

begin

if rising_edge(clk_i) then

if en_i = '1' then

idx := to_integer(unsigned(addr_i(7 downto 0)));

q_r <= ROM_INIT(idx);

end if;

end if;

end process;

end architecture;

stack7x16.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity stack7x16 is

port (

clk_i   : in  std_logic;

rst_i   : in  std_logic;

push_i  : in  std_logic;

pop_i   : in  std_logic;

din_i   : in  word_t;

dout_o  : out word_t;

sp_o    : out unsigned(2 downto 0);

empty_o : out std_logic;

full_o  : out std_logic

);

end entity;

architecture rtl of stack7x16 is

type stack_mem_t is array (0 to STACK_DEPTH-1) of word_t;

signal mem    : stack_mem_t := (others => (others => '0'));

signal sp_r   : unsigned(2 downto 0) := to_unsigned(STACK_DEPTH, 3);

signal dout_r : word_t := (others => '0');

begin

dout_o  <= dout_r;

sp_o    <= sp_r;

empty_o <= '1' when sp_r = to_unsigned(STACK_DEPTH, 3) else '0';

full_o  <= '1' when sp_r = to_unsigned(0, 3) else '0';

process(clk_i)

variable sp_i : integer;

begin

if rising_edge(clk_i) then

if rst_i = '1' then

mem    <= (others => (others => '0'));

sp_r   <= to_unsigned(STACK_DEPTH, 3);

dout_r <= (others => '0');

else

sp_i := to_integer(sp_r);

if push_i = '1' and pop_i = '0' then

if sp_i > 0 then

mem(sp_i - 1) <= din_i;

sp_r <= to_unsigned(sp_i - 1, 3);

end if;

elsif pop_i = '1' and push_i = '0' then

if sp_i < STACK_DEPTH then

dout_r <= mem(sp_i);

sp_r   <= to_unsigned(sp_i + 1, 3);

end if;

end if;

end if;

end if;

end process;

end architecture;

system_core.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity system_core is

port (

clk_i          : in  std_logic;

rst_i          : in  std_logic;

dma_start_i    : in  std_logic;

dma_valid_i    : in  std_logic;

dma_data_i     : in  word_t;

halt_o         : out std_logic;

dma_done_o     : out std_logic;

dbg_state_o    : out std_logic_vector(4 downto 0);

dbg_pc_o       : out addr_t;

dbg_ir0_o      : out word_t;

dbg_ir1_o      : out word_t;

dbg_r1_o       : out word_t;

dbg_r2_o       : out word_t;

dbg_r3_o       : out word_t;

dbg_r4_o       : out word_t;

dbg_r5_o       : out word_t;

dbg_r6_o       : out word_t;

dbg_r7_o       : out word_t;

dbg_flags_o    : out std_logic_vector(3 downto 0);

dbg_sp_o       : out unsigned(2 downto 0);

dbg_cache_hit_o  : out std_logic;

dbg_cache_miss_o : out std_logic;

dbg_req_cpu_o    : out std_logic;

dbg_req_dma_o    : out std_logic;

dbg_gnt_cpu_o    : out std_logic;

dbg_gnt_dma_o    : out std_logic;

dbg_ram_we_o     : out std_logic;

dbg_ram_addr_o   : out addr_t;

dbg_ram_wdata_o  : out word_t;

dbg_ram_rdata_o  : out word_t;

dbg_bp_hist_o    : out std_logic_vector(1 downto 0);

dbg_bp_pred_o    : out std_logic;

dbg_bp_target_o  : out addr_t

);

end entity;

architecture structural of system_core is

signal rom_addr     : addr_t;

signal rom_en       : std_logic;

signal rom_data     : word_t;

signal cache_req    : std_logic;

signal cache_we     : std_logic;

signal cache_addr   : addr_t;

signal cache_wdata  : word_t;

signal cache_rdata  : word_t;

signal cache_ready  : std_logic;

signal cache_hit    : std_logic;

signal cache_miss   : std_logic;

signal cache_ram_req   : std_logic;

signal cache_ram_we    : std_logic;

signal cache_ram_addr  : addr_t;

signal cache_ram_wdata : word_t;

signal dma_req      : std_logic;

signal dma_grant    : std_logic;

signal cpu_grant    : std_logic;

signal dma_done     : std_logic;

signal dma_we       : std_logic;

signal dma_addr     : addr_t;

signal dma_wdata    : word_t;

signal ram_en       : std_logic;

signal ram_we       : std_logic;

signal ram_addr     : addr_t;

signal ram_wdata    : word_t;

signal ram_rdata    : word_t;

signal bp_query     : std_logic;

signal bp_pc_query  : addr_t;

signal bp_hist      : std_logic_vector(1 downto 0);

signal bp_pred      : std_logic;

signal bp_target    : addr_t;

signal bp_update    : std_logic;

signal bp_pc_update : addr_t;

signal bp_hist_upd  : std_logic_vector(1 downto 0);

signal bp_taken     : std_logic;

signal bp_tgt_upd   : addr_t;

begin

ram_en    <= (cache_ram_req and cpu_grant) or (dma_we and dma_grant);

ram_we    <= dma_we when dma_grant = '1' else cache_ram_we;

ram_addr  <= dma_addr when dma_grant = '1' else cache_ram_addr;

ram_wdata <= dma_wdata when dma_grant = '1' else cache_ram_wdata;

dma_done_o        <= dma_done;

dbg_cache_hit_o   <= cache_hit;

dbg_cache_miss_o  <= cache_miss;

dbg_req_cpu_o     <= cache_ram_req;

dbg_req_dma_o     <= dma_req;

dbg_gnt_cpu_o     <= cpu_grant;

dbg_gnt_dma_o     <= dma_grant;

dbg_ram_we_o      <= ram_we;

dbg_ram_addr_o    <= ram_addr;

dbg_ram_wdata_o   <= ram_wdata;

dbg_ram_rdata_o   <= ram_rdata;

dbg_bp_hist_o     <= bp_hist;

dbg_bp_pred_o     <= bp_pred;

dbg_bp_target_o   <= bp_target;

U_CPU : entity work.cpu_core

port map (

clk_i          => clk_i,

rst_i          => rst_i,

rom_addr_o     => rom_addr,

rom_en_o       => rom_en,

rom_data_i     => rom_data,

cache_req_o    => cache_req,

cache_we_o     => cache_we,

cache_addr_o   => cache_addr,

cache_wdata_o  => cache_wdata,

cache_rdata_i  => cache_rdata,

cache_ready_i  => cache_ready,

cache_hit_i    => cache_hit,

cache_miss_i   => cache_miss,

bp_query_o     => bp_query,

bp_pc_query_o  => bp_pc_query,

bp_hist_i      => bp_hist,

bp_pred_i      => bp_pred,

bp_target_i    => bp_target,

bp_update_o    => bp_update,

bp_pc_update_o => bp_pc_update,

bp_hist_o      => bp_hist_upd,

bp_taken_o     => bp_taken,

bp_target_o    => bp_tgt_upd,

halt_o         => halt_o,

dbg_state_o    => dbg_state_o,

dbg_pc_o       => dbg_pc_o,

dbg_ir0_o      => dbg_ir0_o,

dbg_ir1_o      => dbg_ir1_o,

dbg_r1_o       => dbg_r1_o,

dbg_r2_o       => dbg_r2_o,

dbg_r3_o       => dbg_r3_o,

dbg_r4_o       => dbg_r4_o,

dbg_r5_o       => dbg_r5_o,

dbg_r6_o       => dbg_r6_o,

dbg_r7_o       => dbg_r7_o,

dbg_flags_o    => dbg_flags_o,

dbg_sp_o       => dbg_sp_o

);

U_ROM : entity work.rom_sync

port map (

clk_i  => clk_i,

en_i   => rom_en,

addr_i => rom_addr,

data_o => rom_data

);

U_CACHE : entity work.cache_4way_age

port map (

clk_i       => clk_i,

rst_i       => rst_i,

cpu_req_i   => cache_req,

cpu_we_i    => cache_we,

cpu_addr_i  => cache_addr,

cpu_wdata_i => cache_wdata,

cpu_rdata_o => cache_rdata,

cpu_ready_o => cache_ready,

hit_o       => cache_hit,

miss_o      => cache_miss,

ram_req_o   => cache_ram_req,

ram_we_o    => cache_ram_we,

ram_addr_o  => cache_ram_addr,

ram_wdata_o => cache_ram_wdata,

ram_rdata_i => ram_rdata,

ram_grant_i => cpu_grant

);

U_RAM : entity work.ram_sync

port map (

clk_i  => clk_i,

en_i   => ram_en,

we_i   => ram_we,

addr_i => ram_addr,

din_i  => ram_wdata,

dout_o => ram_rdata

);

U_ARB : entity work.bus_arbiter_2master

port map (

req_cpu_i   => cache_ram_req,

req_dma_i   => dma_req,

grant_cpu_o => cpu_grant,

grant_dma_o => dma_grant

);

U_DMA : entity work.dma_controller_3word

port map (

clk_i       => clk_i,

rst_i       => rst_i,

start_i     => dma_start_i,

grant_i     => dma_grant,

dev_valid_i => dma_valid_i,

dev_data_i  => dma_data_i,

req_o       => dma_req,

busy_o      => open,

done_o      => dma_done,

ram_we_o    => dma_we,

ram_addr_o  => dma_addr,

ram_data_o  => dma_wdata

);

U_BP : entity work.branch_predictor

port map (

clk_i          => clk_i,

rst_i          => rst_i,

query_valid_i  => bp_query,

pc_query_i     => bp_pc_query,

hist_o         => bp_hist,

pred_taken_o   => bp_pred,

pred_target_o  => bp_target,

update_valid_i => bp_update,

pc_update_i    => bp_pc_update,

hist_i         => bp_hist_upd,

actual_taken_i => bp_taken,

actual_tgt_i   => bp_tgt_upd

);

end architecture;

Е.2 Проверочные стенды

tb_alu_core.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity tb_alu_core is

end entity;

architecture sim of tb_alu_core is

signal a      : word_t := (others => '0');

signal b      : word_t := (others => '0');

signal sflag  : std_logic := '0';

signal op     : std_logic_vector(2 downto 0) := ALU_PASS_A;

signal y      : word_t;

signal z, s, c, o : std_logic;

begin

DUT : entity work.alu_core

port map (

a_i => a,

b_i => b,

flag_s_i => sflag,

op_i => op,

y_o => y,

z_o => z,

s_o => s,

c_o => c,

o_o => o

);

stimulus : process

begin

a <= x"8001";

b <= x"00F0";

op <= ALU_SRA;

wait for 10 ns;

assert y = x"C000" and c = '1' and s = '1'

report "SRA 8001h must produce C000h, C=1, S=1" severity failure;

a <= y;

sflag <= '1';

op <= ALU_INCS;

wait for 10 ns;

assert y = x"C001"

report "INCS with S=1 must increment C000h to C001h" severity failure;

a <= x"00F0";

b <= x"0F0F";

op <= ALU_OR;

wait for 10 ns;

assert y = x"0FFF"

report "OR must produce 0FFFh" severity failure;

a <= x"0FFF";

b <= x"0FFF";

op <= ALU_NOR;

wait for 10 ns;

assert y = x"F000"

report "NOR must produce F000h" severity failure;

sflag <= '0';

a <= x"1234";

op <= ALU_INCS;

wait for 10 ns;

assert y = x"1234"

report "INCS with S=0 must keep source value" severity failure;

assert false report "tb_alu_core: TEST PASSED" severity note;

wait;

end process;

end architecture;

tb_mem_dma.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity tb_mem_dma is

end entity;

architecture sim of tb_mem_dma is

signal clk      : std_logic := '0';

signal rst      : std_logic := '1';

signal start    : std_logic := '0';

signal dev_valid: std_logic := '0';

signal dev_data : word_t    := (others => '0');

signal dma_req  : std_logic;

signal dma_gnt  : std_logic;

signal dma_done : std_logic;

signal ram_we   : std_logic;

signal ram_addr : addr_t;

signal ram_din  : word_t;

signal ram_dout : word_t;

begin

clk <= not clk after 5 ns;

dma_gnt <= dma_req;

U_DMA : entity work.dma_controller_3word

port map (

clk_i => clk,

rst_i => rst,

start_i => start,

grant_i => dma_gnt,

dev_valid_i => dev_valid,

dev_data_i => dev_data,

req_o => dma_req,

busy_o => open,

done_o => dma_done,

ram_we_o => ram_we,

ram_addr_o => ram_addr,

ram_data_o => ram_din

);

U_RAM : entity work.ram_sync

port map (

clk_i => clk,

en_i => '1',

we_i => ram_we,

addr_i => ram_addr,

din_i => ram_din,

dout_o => ram_dout

);

monitor : process(clk)

variable write_count : integer := 0;

begin

if rising_edge(clk) then

if rst = '1' then

write_count := 0;

elsif ram_we = '1' then

case write_count is

when 0 =>

assert ram_addr = x"000A" and ram_din = x"1111"

report "DMA first write must target 000Ah with 1111h" severity failure;

when 1 =>

assert ram_addr = x"000B" and ram_din = x"2222"

report "DMA second write must target 000Bh with 2222h" severity failure;

when 2 =>

assert ram_addr = x"000C" and ram_din = x"3333"

report "DMA third write must target 000Ch with 3333h" severity failure;

when others =>

assert false report "Unexpected extra DMA write" severity failure;

end case;

write_count := write_count + 1;

end if;

end if;

end process;

stimulus : process

begin

wait for 30 ns;

rst <= '0';

wait until rising_edge(clk);

start    <= '1';

dev_valid <= '1';

dev_data <= x"1111";

wait until rising_edge(clk);

start <= '0';

wait until rising_edge(clk);

dev_data <= x"1111";

wait until rising_edge(clk);

dev_data <= x"2222";

wait until rising_edge(clk);

dev_data <= x"3333";

wait until rising_edge(clk);

dev_valid <= '0';

wait until rising_edge(clk);

wait for 1 ns;

assert dma_done = '1'

report "DMA must assert done after three words" severity failure;

assert false report "tb_mem_dma: TEST PASSED" severity note;

wait;

end process;

end architecture;

tb_stack7x16.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity tb_stack7x16 is

end entity;

architecture sim of tb_stack7x16 is

signal clk   : std_logic := '0';

signal rst   : std_logic := '1';

signal push  : std_logic := '0';

signal pop   : std_logic := '0';

signal din   : word_t := (others => '0');

signal dout  : word_t;

signal sp    : unsigned(2 downto 0);

signal empty : std_logic;

signal full  : std_logic;

begin

clk <= not clk after 5 ns;

DUT : entity work.stack7x16

port map (

clk_i => clk,

rst_i => rst,

push_i => push,

pop_i => pop,

din_i => din,

dout_o => dout,

sp_o => sp,

empty_o => empty,

full_o => full

);

stimulus : process

begin

wait for 12 ns;

rst <= '0';

wait for 1 ns;

assert sp = to_unsigned(7, 3) and empty = '1' and full = '0'

report "After reset stack must be empty with SP=7" severity failure;

din <= x"0002"; push <= '1'; wait for 10 ns; push <= '0';

din <= x"0006"; push <= '1'; wait for 10 ns; push <= '0';

din <= x"0007"; push <= '1'; wait for 10 ns; push <= '0';

din <= x"0078"; push <= '1'; wait for 10 ns; push <= '0';

din <= x"03E0"; push <= '1'; wait for 10 ns; push <= '0';

wait for 1 ns;

assert sp = to_unsigned(2, 3)

report "After five pushes SP must be 2" severity failure;

pop <= '1'; wait for 10 ns; pop <= '0';

wait for 1 ns;

assert dout = x"03E0"

report "First pop must return last pushed value 03E0h" severity failure;

pop <= '1'; wait for 10 ns; pop <= '0';

pop <= '1'; wait for 10 ns; pop <= '0';

pop <= '1'; wait for 10 ns; pop <= '0';

pop <= '1'; wait for 10 ns; pop <= '0';

wait for 1 ns;

assert sp = to_unsigned(7, 3) and empty = '1'

report "After five pops stack must be empty with SP=7" severity failure;

assert false report "tb_stack7x16: TEST PASSED" severity note;

wait;

end process;

end architecture;

tb_system.vhd

library ieee;

use ieee.std_logic_1164.all;

use ieee.numeric_std.all;

use work.microcomputer_pkg.all;

entity tb_system is

end entity;

architecture sim of tb_system is

signal clk          : std_logic := '0';

signal rst          : std_logic := '1';

signal dma_start    : std_logic := '0';

signal dma_valid    : std_logic := '0';

signal dma_data     : word_t := (others => '0');

signal halt         : std_logic;

signal dma_done     : std_logic;

signal dbg_state    : std_logic_vector(4 downto 0);

signal dbg_pc       : addr_t;

signal dbg_ir0      : word_t;

signal dbg_ir1      : word_t;

signal dbg_r1       : word_t;

signal dbg_r2       : word_t;

signal dbg_r3       : word_t;

signal dbg_r4       : word_t;

signal dbg_r5       : word_t;

signal dbg_r6       : word_t;

signal dbg_r7       : word_t;

signal dbg_flags    : std_logic_vector(3 downto 0);

signal dbg_sp       : unsigned(2 downto 0);

signal cache_hit    : std_logic;

signal cache_miss   : std_logic;

signal req_cpu      : std_logic;

signal req_dma      : std_logic;

signal gnt_cpu      : std_logic;

signal gnt_dma      : std_logic;

signal ram_we       : std_logic;

signal ram_addr     : addr_t;

signal ram_wdata    : word_t;

signal ram_rdata    : word_t;

signal bp_hist      : std_logic_vector(1 downto 0);

signal bp_pred      : std_logic;

signal bp_target    : addr_t;

begin

clk <= not clk after 5 ns;

UUT : entity work.system_core

port map (

clk_i            => clk,

rst_i            => rst,

dma_start_i      => dma_start,

dma_valid_i      => dma_valid,

dma_data_i       => dma_data,

halt_o           => halt,

dma_done_o       => dma_done,

dbg_state_o      => dbg_state,

dbg_pc_o         => dbg_pc,

dbg_ir0_o        => dbg_ir0,

dbg_ir1_o        => dbg_ir1,

dbg_r1_o         => dbg_r1,

dbg_r2_o         => dbg_r2,

dbg_r3_o         => dbg_r3,

dbg_r4_o         => dbg_r4,

dbg_r5_o         => dbg_r5,

dbg_r6_o         => dbg_r6,

dbg_r7_o         => dbg_r7,

dbg_flags_o      => dbg_flags,

dbg_sp_o         => dbg_sp,

dbg_cache_hit_o  => cache_hit,

dbg_cache_miss_o => cache_miss,

dbg_req_cpu_o    => req_cpu,

dbg_req_dma_o    => req_dma,

dbg_gnt_cpu_o    => gnt_cpu,

dbg_gnt_dma_o    => gnt_dma,

dbg_ram_we_o     => ram_we,

dbg_ram_addr_o   => ram_addr,

dbg_ram_wdata_o  => ram_wdata,

dbg_ram_rdata_o  => ram_rdata,

dbg_bp_hist_o    => bp_hist,

dbg_bp_pred_o    => bp_pred,

dbg_bp_target_o  => bp_target

);

stim : process

begin

rst <= '1';

wait for 30 ns;

rst <= '0';

wait until rising_edge(clk);

wait until rising_edge(clk);

dma_start <= '1';

dma_valid <= '1';

dma_data  <= x"1111";

wait until rising_edge(clk);

dma_start <= '0';

wait until rising_edge(clk);

dma_data <= x"1111";

wait until rising_edge(clk);

dma_data <= x"2222";

wait until rising_edge(clk);

dma_data <= x"3333";

wait until rising_edge(clk);

dma_valid <= '0';

wait until halt = '1';

wait for 20 ns;

assert dbg_r1 = x"C001" report "R1 must be C001 after SRA + INCS" severity failure;

assert dbg_r2 = x"F000" report "R2 must be F000 after OR + NOR" severity failure;

assert dbg_r3 = x"C001" report "R3 must receive popped stack value" severity failure;

assert dbg_r4 = x"0000" report "R4 must be zero after NOR FFFF,FFFF" severity failure;

assert dbg_r5 = x"0000" report "R5 must remain zero because JZ skips 0018h" severity failure;

assert dbg_r6 = x"0000" report "R6 must remain zero because JMP skips 001Ch" severity failure;

assert dbg_r7 = x"1111" report "R7 must read first DMA word from RAM[000Ah]" severity failure;

assert dbg_flags = "1000" report "Final flags must be Z=1, S=0, C=0, O=0" severity failure;

assert dbg_sp = to_unsigned(7, 3) report "Stack pointer must return to empty value 7" severity failure;

assert dma_done = '1' report "DMA must complete three-word transfer" severity failure;

assert bp_hist = "01" report "GHR must become 01 after one taken JZ" severity failure;

assert false report "tb_system: TEST PASSED" severity note;

wait;

end process;

end architecture;