Министерство образования Республики Беларусь

Учреждение образования

Белорусский государственный университет информатики и радиоэлектроники

Кафедра электронных вычислительных машин

ПРИЛОЖЕНИЕ В

VHDL-листинги

Студент: Власов

Руководитель: ____________________

Минск 2026

# src/alu_core.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity alu_core is
  port (
    a_i      : in  word_t;
    b_i      : in  word_t;
    flag_s_i : in  std_logic;
    op_i     : in  std_logic_vector(2 downto 0);
    y_o      : out word_t;
    z_o      : out std_logic;
    s_o      : out std_logic;
    c_o      : out std_logic;
    o_o      : out std_logic
  );
end entity;

architecture rtl of alu_core is
begin
  process(a_i, b_i, flag_s_i, op_i)
    variable a_u   : unsigned(15 downto 0);
    variable sum17 : unsigned(16 downto 0);
    variable y_v   : word_t;
    variable z_v   : std_logic;
    variable s_v   : std_logic;
    variable c_v   : std_logic;
    variable o_v   : std_logic;
  begin
    a_u := unsigned(a_i);
    y_v := a_i;
    c_v := '0';
    o_v := '0';

    case op_i is
      when ALU_OR =>
        y_v := a_i or b_i;
      when ALU_NOR =>
        y_v := not (a_i or b_i);
      when ALU_SRA =>
        y_v := a_i(15) & a_i(15 downto 1);
        c_v := a_i(0);
      when ALU_INCS =>
        sum17 := ('0' & a_u);
        if flag_s_i = '1' then
          sum17 := ('0' & a_u) + to_unsigned(1, 17);
        end if;
        y_v := std_logic_vector(sum17(15 downto 0));
        c_v := sum17(16);
        o_v := (not a_i(15)) and flag_s_i and y_v(15);
      when others =>
        y_v := a_i;
    end case;

    if y_v = x"0000" then
      z_v := '1';
    else
      z_v := '0';
    end if;
    s_v := y_v(15);

    y_o <= y_v;
    z_o <= z_v;
    s_o <= s_v;
    c_o <= c_v;
    o_o <= o_v;
  end process;
end architecture;

# src/branch_predictor.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity branch_predictor is
  port (
    clk_i          : in  std_logic;
    rst_i          : in  std_logic;
    query_valid_i  : in  std_logic;
    pc_query_i     : in  addr_t;
    hist_o         : out std_logic_vector(1 downto 0);
    pred_taken_o   : out std_logic;
    pred_target_o  : out addr_t;
    update_valid_i : in  std_logic;
    pc_update_i    : in  addr_t;
    hist_i         : in  std_logic_vector(1 downto 0);
    actual_taken_i : in  std_logic;
    actual_tgt_i   : in  addr_t
  );
end entity;

architecture rtl of branch_predictor is
  type pht_t   is array (0 to 15) of unsigned(1 downto 0);
  type target_t is array (0 to 15) of addr_t;
  type valid_t  is array (0 to 15) of std_logic;

  signal pht       : pht_t    := (others => "01");
  signal btb_tgt   : target_t := (others => (others => '0'));
  signal btb_valid : valid_t  := (others => '0');
  signal ghr_r     : std_logic_vector(1 downto 0) := "00";

  function idx_of(pc : addr_t; hist : std_logic_vector(1 downto 0)) return integer is
    variable idx_v : std_logic_vector(3 downto 0);
  begin
    idx_v := std_logic_vector(pc(1 downto 0)) & hist;
    return to_integer(unsigned(idx_v));
  end function;

  function sat_dec(v : unsigned(1 downto 0)) return unsigned is
  begin
    if v = "00" then
      return v;
    else
      return v - 1;
    end if;
  end function;
begin
  hist_o <= ghr_r;

  process(query_valid_i, pc_query_i, ghr_r, pht, btb_valid, btb_tgt)
    variable idx : integer;
  begin
    pred_taken_o  <= '0';
    pred_target_o <= (others => '0');
    if query_valid_i = '1' then
      idx := idx_of(pc_query_i, ghr_r);
      if pht(idx)(1) = '1' and btb_valid(idx) = '1' then
        pred_taken_o  <= '1';
        pred_target_o <= btb_tgt(idx);
      end if;
    end if;
  end process;

  process(clk_i)
    variable idx : integer;
  begin
    if rising_edge(clk_i) then
      if rst_i = '1' then
        pht       <= (others => "01");
        btb_tgt   <= (others => (others => '0'));
        btb_valid <= (others => '0');
        ghr_r     <= "00";
      elsif update_valid_i = '1' then
        idx := idx_of(pc_update_i, hist_i);
        if actual_taken_i = '1' then
          pht(idx)       <= sat_inc2(pht(idx));
          btb_tgt(idx)   <= actual_tgt_i;
          btb_valid(idx) <= '1';
        else
          pht(idx) <= sat_dec(pht(idx));
        end if;
        ghr_r <= ghr_r(0) & actual_taken_i;
      end if;
    end if;
  end process;
end architecture;

# src/bus_arbiter_2master.vhd

library ieee;
use ieee.std_logic_1164.all;

entity bus_arbiter_2master is
  port (
    req_cpu_i   : in  std_logic;
    req_dma_i   : in  std_logic;
    grant_cpu_o : out std_logic;
    grant_dma_o : out std_logic
  );
end entity;

architecture rtl of bus_arbiter_2master is
begin
  grant_dma_o <= req_dma_i;
  grant_cpu_o <= req_cpu_i and (not req_dma_i);
end architecture;

# src/cache_4way_age.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity cache_4way_age is
  port (
    clk_i        : in  std_logic;
    rst_i        : in  std_logic;

    cpu_req_i    : in  std_logic;
    cpu_we_i     : in  std_logic;
    cpu_addr_i   : in  addr_t;
    cpu_wdata_i  : in  word_t;
    cpu_rdata_o  : out word_t;
    cpu_ready_o  : out std_logic;
    hit_o        : out std_logic;
    miss_o       : out std_logic;

    ram_req_o    : out std_logic;
    ram_we_o     : out std_logic;
    ram_addr_o   : out addr_t;
    ram_wdata_o  : out word_t;
    ram_rdata_i  : in  word_t;
    ram_grant_i  : in  std_logic
  );
end entity;

architecture rtl of cache_4way_age is
  subtype cache_tag_t is std_logic_vector(11 downto 0);
  type cache_state_t is (IDLE, WAIT_READ_GRANT, WAIT_READ_DATA, WAIT_WRITE_GRANT);
  type set_way_word_t is array (0 to CACHE_SETS-1, 0 to CACHE_WAYS-1) of word_t;
  type set_way_tag_t is array (0 to CACHE_SETS-1, 0 to CACHE_WAYS-1) of cache_tag_t;
  type set_way_age_t is array (0 to CACHE_SETS-1, 0 to CACHE_WAYS-1) of unsigned(1 downto 0);
  type set_way_valid_t is array (0 to CACHE_SETS-1, 0 to CACHE_WAYS-1) of std_logic;

  signal state_r      : cache_state_t := IDLE;
  signal data_r       : set_way_word_t := (others => (others => (others => '0')));
  signal tag_r        : set_way_tag_t := (others => (others => (others => '0')));
  signal age_r        : set_way_age_t := (others => (others => (others => '0')));
  signal valid_r      : set_way_valid_t := (others => (others => '0'));

  signal req_addr_r   : addr_t := (others => '0');
  signal req_wdata_r  : word_t := (others => '0');
  signal req_we_r     : std_logic := '0';
  signal write_hit_r  : std_logic := '0';
  signal victim_r     : integer range 0 to CACHE_WAYS-1 := 0;
  signal set_r        : integer range 0 to CACHE_SETS-1 := 0;
  signal tag_req_r    : cache_tag_t := (others => '0');

  signal cpu_rdata_r  : word_t := (others => '0');
  signal cpu_ready_r  : std_logic := '0';
  signal hit_r        : std_logic := '0';
  signal miss_r       : std_logic := '0';
  signal ram_req_r    : std_logic := '0';
  signal ram_we_r     : std_logic := '0';
  signal ram_addr_r   : addr_t := (others => '0');
  signal ram_wdata_r  : word_t := (others => '0');

  function cache_set(addr : addr_t) return integer is
  begin
    return to_integer(unsigned(addr(3 downto 0)));
  end function;

  function cache_tag(addr : addr_t) return cache_tag_t is
  begin
    return addr(15 downto 4);
  end function;

  function inc_age(v : unsigned(1 downto 0)) return unsigned is
  begin
    if v = "11" then
      return v;
    else
      return v + 1;
    end if;
  end function;
begin
  cpu_rdata_o <= cpu_rdata_r;
  cpu_ready_o <= cpu_ready_r;

hit_o       <= hit_r;
  miss_o      <= miss_r;
  ram_req_o   <= ram_req_r;
  ram_we_o    <= ram_we_r;
  ram_addr_o  <= ram_addr_r;
  ram_wdata_o <= ram_wdata_r;

  process(clk_i)
    variable set_v    : integer range 0 to CACHE_SETS-1;
    variable hit_v    : boolean;
    variable hit_way  : integer range 0 to CACHE_WAYS-1;
    variable victim_v : integer range 0 to CACHE_WAYS-1;
    variable tag_v    : std_logic_vector(11 downto 0);
  begin
    if rising_edge(clk_i) then
      if rst_i = '1' then
        state_r     <= IDLE;
        data_r      <= (others => (others => (others => '0')));
        tag_r       <= (others => (others => (others => '0')));
        age_r       <= (others => (others => (others => '0')));
        valid_r     <= (others => (others => '0'));
        req_addr_r  <= (others => '0');
        req_wdata_r <= (others => '0');
        req_we_r    <= '0';
        write_hit_r <= '0';
        victim_r    <= 0;
        set_r       <= 0;
        tag_req_r   <= (others => '0');
        cpu_rdata_r <= (others => '0');
        cpu_ready_r <= '0';
        hit_r       <= '0';
        miss_r      <= '0';
        ram_req_r   <= '0';
        ram_we_r    <= '0';
        ram_addr_r  <= (others => '0');
        ram_wdata_r <= (others => '0');
      else
        cpu_ready_r <= '0';
        hit_r       <= '0';
        miss_r      <= '0';

        case state_r is
          when IDLE =>
            ram_req_r <= '0';
            ram_we_r  <= '0';

            if cpu_req_i = '1' and cpu_ready_r = '0' then
              set_v := cache_set(cpu_addr_i);
              tag_v := cache_tag(cpu_addr_i);
              hit_v := false;
              hit_way := 0;
              victim_v := 0;

              for way in 0 to CACHE_WAYS-1 loop
                if valid_r(set_v, way) = '1' and tag_r(set_v, way) = tag_v then
                  hit_v := true;
                  hit_way := way;
                end if;
              end loop;

              for way in 0 to CACHE_WAYS-1 loop
                if valid_r(set_v, way) = '0' then
                  victim_v := way;
                elsif valid_r(set_v, victim_v) = '1' and age_r(set_v, way) > age_r(set_v, victim_v) then
                  victim_v := way;
                end if;
              end loop;

              req_addr_r  <= cpu_addr_i;
              req_wdata_r <= cpu_wdata_i;
              req_we_r    <= cpu_we_i;
              write_hit_r <= '0';
              victim_r    <= victim_v;
              set_r       <= set_v;
              tag_req_r   <= tag_v;

              if hit_v and cpu_we_i = '0' then
                cpu_rdata_r <= data_r(set_v, hit_way);
                cpu_ready_r <= '1';
                hit_r       <= '1';
              elsif cpu_we_i = '1' then
                if hit_v then

data_r(set_v, hit_way) <= cpu_wdata_i;
                  victim_v := hit_way;
                  write_hit_r <= '1';
                end if;
                ram_req_r   <= '1';
                ram_we_r    <= '1';
                ram_addr_r  <= cpu_addr_i;
                ram_wdata_r <= cpu_wdata_i;
                victim_r    <= victim_v;
                state_r     <= WAIT_WRITE_GRANT;
                if hit_v then
                  hit_r <= '1';
                else
                  miss_r <= '1';
                end if;
              else
                ram_req_r   <= '1';
                ram_we_r    <= '0';
                ram_addr_r  <= cpu_addr_i;
                ram_wdata_r <= (others => '0');
                state_r     <= WAIT_READ_GRANT;
                miss_r      <= '1';
              end if;
            end if;

          when WAIT_READ_GRANT =>
            ram_req_r <= '1';
            ram_we_r  <= '0';
            ram_addr_r <= req_addr_r;
            if ram_grant_i = '1' then
              state_r <= WAIT_READ_DATA;
            end if;

          when WAIT_READ_DATA =>
            ram_req_r <= '0';
            ram_we_r  <= '0';
            data_r(set_r, victim_r)  <= ram_rdata_i;
            tag_r(set_r, victim_r)   <= tag_req_r;
            valid_r(set_r, victim_r) <= '1';
            age_r(set_r, victim_r)   <= (others => '0');
            for way in 0 to CACHE_WAYS-1 loop
              if way /= victim_r and valid_r(set_r, way) = '1' then
                age_r(set_r, way) <= inc_age(age_r(set_r, way));
              end if;
            end loop;
            cpu_rdata_r <= ram_rdata_i;
            cpu_ready_r <= '1';
            miss_r      <= '1';
            state_r     <= IDLE;

          when WAIT_WRITE_GRANT =>
            ram_req_r   <= '1';
            ram_we_r    <= '1';
            ram_addr_r  <= req_addr_r;
            ram_wdata_r <= req_wdata_r;
            if ram_grant_i = '1' then
              if write_hit_r = '0' then
                data_r(set_r, victim_r)  <= req_wdata_r;
                tag_r(set_r, victim_r)   <= tag_req_r;
                valid_r(set_r, victim_r) <= '1';
                age_r(set_r, victim_r)   <= (others => '0');
                for way in 0 to CACHE_WAYS-1 loop
                  if way /= victim_r and valid_r(set_r, way) = '1' then
                    age_r(set_r, way) <= inc_age(age_r(set_r, way));
                  end if;
                end loop;
              end if;
              cpu_ready_r <= '1';
              state_r     <= IDLE;
              ram_req_r   <= '0';
              ram_we_r    <= '0';
            end if;
        end case;
      end if;
    end if;
  end process;
end architecture;

# src/cpu_core.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity cpu_core is
  port (
    clk_i          : in  std_logic;
    rst_i          : in  std_logic;

    rom_addr_o     : out addr_t;
    rom_en_o       : out std_logic;
    rom_data_i     : in  word_t;

    cache_req_o    : out std_logic;
    cache_we_o     : out std_logic;
    cache_addr_o   : out addr_t;
    cache_wdata_o  : out word_t;
    cache_rdata_i  : in  word_t;
    cache_ready_i  : in  std_logic;
    cache_hit_i    : in  std_logic;
    cache_miss_i   : in  std_logic;

    bp_query_o     : out std_logic;
    bp_pc_query_o  : out addr_t;
    bp_hist_i      : in  std_logic_vector(1 downto 0);
    bp_pred_i      : in  std_logic;
    bp_target_i    : in  addr_t;
    bp_update_o    : out std_logic;
    bp_pc_update_o : out addr_t;
    bp_hist_o      : out std_logic_vector(1 downto 0);
    bp_taken_o     : out std_logic;
    bp_target_o    : out addr_t;

    halt_o         : out std_logic;

    dbg_state_o    : out std_logic_vector(4 downto 0);
    dbg_pc_o       : out addr_t;
    dbg_ir0_o      : out word_t;
    dbg_ir1_o      : out word_t;
    dbg_r1_o       : out word_t;
    dbg_r2_o       : out word_t;
    dbg_r3_o       : out word_t;
    dbg_r4_o       : out word_t;
    dbg_r5_o       : out word_t;
    dbg_r6_o       : out word_t;
    dbg_r7_o       : out word_t;
    dbg_flags_o    : out std_logic_vector(3 downto 0);
    dbg_sp_o       : out unsigned(2 downto 0)
  );
end entity;

architecture rtl of cpu_core is
  subtype state_code_t is std_logic_vector(4 downto 0);
  type state_t is (
    S_RESET,
    S_FETCH0_REQ,
    S_FETCH0_WAIT,
    S_FETCH0_LATCH,
    S_FETCH1_REQ,
    S_FETCH1_WAIT,
    S_FETCH1_LATCH,
    S_DECODE,
    S_CACHE_READ,
    S_CACHE_WRITE,
    S_ALU_SETUP_REG,
    S_ALU_WRITE,
    S_RF_WRITE_MEM,
    S_STACK_PUSH,
    S_STACK_POP,
    S_RF_WRITE_POP,
    S_BRANCH_JMP,
    S_BRANCH_JZ,
    S_FINISH,
    S_HALT
  );

  signal state_r       : state_t := S_RESET;
  signal pc_r          : addr_t := (others => '0');
  signal ir0_r         : word_t := (others => '0');
  signal ir1_r         : word_t := (others => '0');
  signal cur_reg_r     : reg_idx_t := (others => '0');
  signal cur_opcode_r  : std_logic_vector(7 downto 0) := OP_HLT;

  signal rom_addr_r    : addr_t := (others => '0');
  signal rom_en_r      : std_logic := '0';

  signal rf_we         : std_logic;
  signal rf_wr_addr    : reg_idx_t;
  signal rf_rd_a       : reg_idx_t;
  signal rf_rd_b       : reg_idx_t;
  signal rf_din        : word_t;
  signal rf_a          : word_t;
  signal rf_b          : word_t;

  signal alu_a_r       : word_t := (others => '0');
  signal alu_b_r       : word_t := (others => '0');

signal alu_op_r      : std_logic_vector(2 downto 0) := ALU_PASS_A;
  signal alu_y         : word_t;
  signal alu_z         : std_logic;
  signal alu_s         : std_logic;
  signal alu_c         : std_logic;
  signal alu_o         : std_logic;

  signal flag_we       : std_logic;
  signal flag_z        : std_logic;
  signal flag_s        : std_logic;
  signal flag_c        : std_logic;
  signal flag_o        : std_logic;

  signal stack_push    : std_logic;
  signal stack_pop     : std_logic;
  signal stack_dout    : word_t;
  signal stack_sp      : unsigned(2 downto 0);
  signal stack_empty   : std_logic;
  signal stack_full    : std_logic;

  signal r1_dbg        : word_t;
  signal r2_dbg        : word_t;
  signal r3_dbg        : word_t;
  signal r4_dbg        : word_t;
  signal r5_dbg        : word_t;
  signal r6_dbg        : word_t;
  signal r7_dbg        : word_t;

  function state_code(s : state_t) return state_code_t is
  begin
    case s is
      when S_RESET        => return "00000";
      when S_FETCH0_REQ   => return "00001";
      when S_FETCH0_WAIT  => return "00010";
      when S_FETCH0_LATCH => return "00011";
      when S_FETCH1_REQ   => return "00100";
      when S_FETCH1_WAIT  => return "00101";
      when S_FETCH1_LATCH => return "00110";
      when S_DECODE       => return "00111";
      when S_CACHE_READ   => return "01000";
      when S_CACHE_WRITE  => return "01001";
      when S_ALU_SETUP_REG=> return "01010";
      when S_ALU_WRITE    => return "01011";
      when S_RF_WRITE_MEM => return "01100";
      when S_STACK_PUSH   => return "01101";
      when S_STACK_POP    => return "01110";
      when S_RF_WRITE_POP => return "01111";
      when S_BRANCH_JMP   => return "10000";
      when S_BRANCH_JZ    => return "10001";
      when S_FINISH       => return "10010";
      when S_HALT         => return "11111";
    end case;
  end function;
begin
  rom_addr_o <= rom_addr_r;
  rom_en_o   <= rom_en_r;

  rf_wr_addr <= cur_reg_r;
  rf_rd_a    <= cur_reg_r;
  rf_rd_b    <= (others => '0');
  rf_we      <= '1' when state_r = S_ALU_WRITE or state_r = S_RF_WRITE_MEM or state_r = S_RF_WRITE_POP else '0';
  rf_din     <= alu_y when state_r = S_ALU_WRITE else
                cache_rdata_i when state_r = S_RF_WRITE_MEM else
                stack_dout;

  flag_we <= '1' when state_r = S_ALU_WRITE else '0';

  stack_push <= '1' when state_r = S_STACK_PUSH else '0';
  stack_pop  <= '1' when state_r = S_STACK_POP else '0';

  cache_req_o   <= '1' when state_r = S_CACHE_READ or state_r = S_CACHE_WRITE else '0';
  cache_we_o    <= '1' when state_r = S_CACHE_WRITE else '0';
  cache_addr_o  <= ir1_r;
  cache_wdata_o <= rf_a;

  bp_query_o     <= '1' when state_r = S_FETCH0_REQ else '0';
  bp_pc_query_o  <= pc_r;

bp_update_o    <= '1' when state_r = S_BRANCH_JZ else '0';
  bp_pc_update_o <= pc_r;
  bp_hist_o      <= bp_hist_i;
  bp_taken_o     <= flag_z;
  bp_target_o    <= ir1_r;

  halt_o      <= '1' when state_r = S_HALT else '0';
  dbg_state_o <= state_code(state_r);
  dbg_pc_o    <= pc_r;
  dbg_ir0_o   <= ir0_r;
  dbg_ir1_o   <= ir1_r;
  dbg_r1_o    <= r1_dbg;
  dbg_r2_o    <= r2_dbg;
  dbg_r3_o    <= r3_dbg;
  dbg_r4_o    <= r4_dbg;
  dbg_r5_o    <= r5_dbg;
  dbg_r6_o    <= r6_dbg;
  dbg_r7_o    <= r7_dbg;
  dbg_flags_o <= flag_z & flag_s & flag_c & flag_o;
  dbg_sp_o    <= stack_sp;

  U_RF : entity work.reg_file12x16_dbg
    port map (
      clk_i       => clk_i,
      rst_i       => rst_i,
      we_i        => rf_we,
      wr_addr_i   => rf_wr_addr,
      rd_addr_a_i => rf_rd_a,
      rd_addr_b_i => rf_rd_b,
      din_i       => rf_din,
      dout_a_o    => rf_a,
      dout_b_o    => rf_b,
      r0_o        => open,
      r1_o        => r1_dbg,
      r2_o        => r2_dbg,
      r3_o        => r3_dbg,
      r4_o        => r4_dbg,
      r5_o        => r5_dbg,
      r6_o        => r6_dbg,
      r7_o        => r7_dbg,
      r8_o        => open,
      r9_o        => open,
      r10_o       => open,
      r11_o       => open
    );

  U_ALU : entity work.alu_core
    port map (
      a_i      => alu_a_r,
      b_i      => alu_b_r,
      flag_s_i => flag_s,
      op_i     => alu_op_r,
      y_o      => alu_y,
      z_o      => alu_z,
      s_o      => alu_s,
      c_o      => alu_c,
      o_o      => alu_o
    );

  U_FLAGS : entity work.flags_reg
    port map (
      clk_i => clk_i,
      rst_i => rst_i,
      we_i  => flag_we,
      z_i   => alu_z,
      s_i   => alu_s,
      c_i   => alu_c,
      o_i   => alu_o,
      z_o   => flag_z,
      s_o   => flag_s,
      c_o   => flag_c,
      o_o   => flag_o
    );

  U_STACK : entity work.stack7x16
    port map (
      clk_i   => clk_i,
      rst_i   => rst_i,
      push_i  => stack_push,
      pop_i   => stack_pop,
      din_i   => rf_a,
      dout_o  => stack_dout,
      sp_o    => stack_sp,
      empty_o => stack_empty,
      full_o  => stack_full
    );

  process(clk_i)
    variable opcode_v : std_logic_vector(7 downto 0);
    variable next_pc  : unsigned(15 downto 0);
  begin
    if rising_edge(clk_i) then
      if rst_i = '1' then
        state_r      <= S_RESET;
        pc_r         <= (others => '0');
        ir0_r        <= (others => '0');
        ir1_r        <= (others => '0');
        cur_reg_r    <= (others => '0');
        cur_opcode_r <= OP_HLT;
        rom_addr_r   <= (others => '0');
        rom_en_r     <= '0';
        alu_a_r      <= (others => '0');
        alu_b_r      <= (others => '0');
        alu_op_r     <= ALU_PASS_A;
      else
        rom_en_r <= '1';

next_pc := unsigned(pc_r) + 2;

        case state_r is
          when S_RESET =>
            pc_r       <= (others => '0');
            rom_addr_r <= (others => '0');
            state_r    <= S_FETCH0_REQ;

          when S_FETCH0_REQ =>
            rom_addr_r <= pc_r;
            state_r    <= S_FETCH0_WAIT;

          when S_FETCH0_WAIT =>
            state_r <= S_FETCH0_LATCH;

          when S_FETCH0_LATCH =>
            ir0_r      <= rom_data_i;
            rom_addr_r <= std_logic_vector(unsigned(pc_r) + 1);
            state_r    <= S_FETCH1_REQ;

          when S_FETCH1_REQ =>
            state_r <= S_FETCH1_WAIT;

          when S_FETCH1_WAIT =>
            state_r <= S_FETCH1_LATCH;

          when S_FETCH1_LATCH =>
            ir1_r   <= rom_data_i;
            state_r <= S_DECODE;

          when S_DECODE =>
            opcode_v     := ir0_r(15 downto 8);
            cur_opcode_r <= opcode_v;
            cur_reg_r    <= unsigned(ir0_r(7 downto 4));
            if opcode_v = OP_MOV_MR or opcode_v = OP_OR or opcode_v = OP_NOR then
              state_r <= S_CACHE_READ;
            elsif opcode_v = OP_MOV_RM then
              state_r <= S_CACHE_WRITE;
            elsif opcode_v = OP_SRA or opcode_v = OP_INCS then
              state_r <= S_ALU_SETUP_REG;
            elsif opcode_v = OP_PUSH then
              state_r <= S_STACK_PUSH;
            elsif opcode_v = OP_POP then
              state_r <= S_STACK_POP;
            elsif opcode_v = OP_JMP then
              state_r <= S_BRANCH_JMP;
            elsif opcode_v = OP_JZ then
              state_r <= S_BRANCH_JZ;
            elsif opcode_v = OP_HLT then
              state_r <= S_HALT;
            else
              state_r <= S_HALT;
            end if;

          when S_CACHE_READ =>
            if cache_ready_i = '1' then
              if cur_opcode_r = OP_MOV_MR then
                state_r <= S_RF_WRITE_MEM;
              else
                alu_a_r <= rf_a;
                alu_b_r <= cache_rdata_i;
                if cur_opcode_r = OP_OR then
                  alu_op_r <= ALU_OR;
                else
                  alu_op_r <= ALU_NOR;
                end if;
                state_r <= S_ALU_WRITE;
              end if;
            end if;

          when S_CACHE_WRITE =>
            if cache_ready_i = '1' then
              state_r <= S_FINISH;
            end if;

          when S_ALU_SETUP_REG =>
            alu_a_r <= rf_a;
            alu_b_r <= (others => '0');
            if cur_opcode_r = OP_SRA then
              alu_op_r <= ALU_SRA;
            else
              alu_op_r <= ALU_INCS;
            end if;
            state_r <= S_ALU_WRITE;

          when S_ALU_WRITE =>
            state_r <= S_FINISH;

          when S_RF_WRITE_MEM =>

state_r <= S_FINISH;

          when S_STACK_PUSH =>
            state_r <= S_FINISH;

          when S_STACK_POP =>
            state_r <= S_RF_WRITE_POP;

          when S_RF_WRITE_POP =>
            state_r <= S_FINISH;

          when S_BRANCH_JMP =>
            pc_r    <= ir1_r;
            state_r <= S_FETCH0_REQ;

          when S_BRANCH_JZ =>
            if flag_z = '1' then
              pc_r <= ir1_r;
            else
              pc_r <= std_logic_vector(next_pc);
            end if;
            state_r <= S_FETCH0_REQ;

          when S_FINISH =>
            pc_r    <= std_logic_vector(next_pc);
            state_r <= S_FETCH0_REQ;

          when S_HALT =>
            state_r <= S_HALT;
        end case;
      end if;
    end if;
  end process;

end architecture;

# src/data_image_pkg.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

package data_image_pkg is
  constant RAM_INIT : ram256_t := (
    16#0A# => x"0000",
    16#0B# => x"0000",
    16#0C# => x"0000",
    16#20# => x"8001",
    16#21# => x"00F0",
    16#22# => x"0F0F",
    16#23# => x"0000",
    16#24# => x"1234",
    16#25# => x"FFFF",
    16#30# => x"0000",
    others  => (others => '0')
  );
end package;

# src/dma_controller_3word.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity dma_controller_3word is
  generic (
    BASE_ADDR_G : addr_t := x"000A"
  );
  port (
    clk_i       : in  std_logic;
    rst_i       : in  std_logic;
    start_i     : in  std_logic;
    grant_i     : in  std_logic;
    dev_valid_i : in  std_logic;
    dev_data_i  : in  word_t;
    req_o       : out std_logic;
    busy_o      : out std_logic;
    done_o      : out std_logic;
    ram_we_o    : out std_logic;
    ram_addr_o  : out addr_t;
    ram_data_o  : out word_t
  );
end entity;

architecture rtl of dma_controller_3word is
  type state_t is (IDLE, REQ_BUS, WRITE_WORD, FINISH);
  signal state_r  : state_t := IDLE;
  signal index_r  : unsigned(1 downto 0) := (others => '0');
  signal done_r   : std_logic := '0';
begin
  busy_o     <= '0' when state_r = IDLE else '1';
  req_o      <= '1' when state_r = REQ_BUS or state_r = WRITE_WORD else '0';
  ram_we_o   <= '1' when state_r = WRITE_WORD and grant_i = '1' and dev_valid_i = '1' else '0';
  ram_addr_o <= std_logic_vector(unsigned(BASE_ADDR_G) + resize(index_r, ADDR_W));
  ram_data_o <= dev_data_i;
  done_o     <= done_r;

  process(clk_i)
  begin
    if rising_edge(clk_i) then
      if rst_i = '1' then
        state_r <= IDLE;
        index_r <= (others => '0');
        done_r  <= '0';
      else
        case state_r is
          when IDLE =>
            if start_i = '1' then
              index_r <= (others => '0');
              done_r  <= '0';
              state_r <= REQ_BUS;
            end if;
          when REQ_BUS =>
            if grant_i = '1' then
              state_r <= WRITE_WORD;
            end if;
          when WRITE_WORD =>
            if grant_i = '1' and dev_valid_i = '1' then
              if index_r = "10" then
                state_r <= FINISH;
              else
                index_r <= index_r + 1;
              end if;
            end if;
          when FINISH =>
            done_r  <= '1';
            state_r <= IDLE;
        end case;
      end if;
    end if;
  end process;
end architecture;

# src/flags_reg.vhd

library ieee;
use ieee.std_logic_1164.all;

entity flags_reg is
  port (
    clk_i : in  std_logic;
    rst_i : in  std_logic;
    we_i  : in  std_logic;
    z_i   : in  std_logic;
    s_i   : in  std_logic;
    c_i   : in  std_logic;
    o_i   : in  std_logic;
    z_o   : out std_logic;
    s_o   : out std_logic;
    c_o   : out std_logic;
    o_o   : out std_logic
  );
end entity;

architecture rtl of flags_reg is
  signal z_r, s_r, c_r, o_r : std_logic := '0';
begin
  z_o <= z_r;
  s_o <= s_r;
  c_o <= c_r;
  o_o <= o_r;

  process(clk_i)
  begin
    if rising_edge(clk_i) then
      if rst_i = '1' then
        z_r <= '0';
        s_r <= '0';
        c_r <= '0';
        o_r <= '0';
      elsif we_i = '1' then
        z_r <= z_i;
        s_r <= s_i;
        c_r <= c_i;
        o_r <= o_i;
      end if;
    end if;
  end process;
end architecture;

# src/microcomputer_pkg.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;

package microcomputer_pkg is
  constant DATA_W      : positive := 16;
  constant ADDR_W      : positive := 16;
  constant REG_COUNT   : positive := 12;
  constant STACK_DEPTH : positive := 7;
  constant CACHE_SETS  : positive := 16;
  constant CACHE_WAYS  : positive := 4;

  subtype word_t    is std_logic_vector(DATA_W-1 downto 0);
  subtype addr_t    is std_logic_vector(ADDR_W-1 downto 0);
  subtype reg_idx_t is unsigned(3 downto 0);

  type ram256_t is array (0 to 255) of word_t;

  constant ALU_PASS_A : std_logic_vector(2 downto 0) := "000";
  constant ALU_OR     : std_logic_vector(2 downto 0) := "001";
  constant ALU_NOR    : std_logic_vector(2 downto 0) := "010";
  constant ALU_SRA    : std_logic_vector(2 downto 0) := "011";
  constant ALU_INCS   : std_logic_vector(2 downto 0) := "100";

  constant OP_HLT     : std_logic_vector(7 downto 0) := x"00";
  constant OP_MOV_MR  : std_logic_vector(7 downto 0) := x"01";
  constant OP_MOV_RM  : std_logic_vector(7 downto 0) := x"02";
  constant OP_OR      : std_logic_vector(7 downto 0) := x"03";
  constant OP_NOR     : std_logic_vector(7 downto 0) := x"04";
  constant OP_SRA     : std_logic_vector(7 downto 0) := x"05";
  constant OP_INCS    : std_logic_vector(7 downto 0) := x"06";
  constant OP_PUSH    : std_logic_vector(7 downto 0) := x"07";
  constant OP_POP     : std_logic_vector(7 downto 0) := x"08";
  constant OP_JMP     : std_logic_vector(7 downto 0) := x"09";
  constant OP_JZ      : std_logic_vector(7 downto 0) := x"0A";

  function sat_inc2(v : unsigned(1 downto 0)) return unsigned;
end package;

package body microcomputer_pkg is
  function sat_inc2(v : unsigned(1 downto 0)) return unsigned is
  begin
    if v = "11" then
      return v;
    else
      return v + 1;
    end if;
  end function;
end package body;

# src/microcomputer_top.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity microcomputer_top is
  port (
    CLK              : in  std_logic;
    RESET            : in  std_logic;
    DMA_START        : in  std_logic;
    DMA_VALID        : in  std_logic;
    DMA_DATA         : in  word_t;

    HALT             : out std_logic;
    DMA_DONE         : out std_logic
  );
end entity;

architecture structural of microcomputer_top is
begin
  U_SYSTEM : entity work.system_core
    port map (
      clk_i            => CLK,
      rst_i            => RESET,
      dma_start_i      => DMA_START,
      dma_valid_i      => DMA_VALID,
      dma_data_i       => DMA_DATA,
      halt_o           => HALT,
      dma_done_o       => DMA_DONE,
      dbg_state_o      => open,
      dbg_pc_o         => open,
      dbg_ir0_o        => open,
      dbg_ir1_o        => open,
      dbg_r1_o         => open,
      dbg_r2_o         => open,
      dbg_r3_o         => open,
      dbg_r4_o         => open,
      dbg_r5_o         => open,
      dbg_r6_o         => open,
      dbg_r7_o         => open,
      dbg_flags_o      => open,
      dbg_sp_o         => open,
      dbg_cache_hit_o  => open,
      dbg_cache_miss_o => open,
      dbg_req_cpu_o    => open,
      dbg_req_dma_o    => open,
      dbg_gnt_cpu_o    => open,
      dbg_gnt_dma_o    => open,
      dbg_ram_we_o     => open,
      dbg_ram_addr_o   => open,
      dbg_ram_wdata_o  => open,
      dbg_ram_rdata_o  => open,
      dbg_bp_hist_o    => open,
      dbg_bp_pred_o    => open,
      dbg_bp_target_o  => open
    );
end architecture;

# src/program_image_pkg.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

package program_image_pkg is
  constant ROM_INIT : ram256_t := (
    16#00# => x"0110", -- MOV 0020h, R1
    16#01# => x"0020",
    16#02# => x"0510", -- SRA R1
    16#03# => x"0000",
    16#04# => x"0610", -- INCS R1
    16#05# => x"0000",
    16#06# => x"0710", -- PUSH R1
    16#07# => x"0000",
    16#08# => x"0120", -- MOV 0021h, R2
    16#09# => x"0021",
    16#0A# => x"0320", -- OR R2, 0022h
    16#0B# => x"0022",
    16#0C# => x"0420", -- NOR R2, 0023h
    16#0D# => x"0023",
    16#0E# => x"0830", -- POP R3
    16#0F# => x"0000",
    16#10# => x"0230", -- MOV R3, 0030h
    16#11# => x"0030",
    16#12# => x"0140", -- MOV 0025h, R4
    16#13# => x"0025",
    16#14# => x"0440", -- NOR R4, 0025h
    16#15# => x"0025",
    16#16# => x"0A00", -- JZ 001Ah
    16#17# => x"001A",
    16#18# => x"0150", -- MOV 0024h, R5 (must be skipped)
    16#19# => x"0024",
    16#1A# => x"0900", -- JMP 001Eh
    16#1B# => x"001E",
    16#1C# => x"0160", -- MOV 0024h, R6 (must be skipped)
    16#1D# => x"0024",
    16#1E# => x"0170", -- MOV 000Ah, R7
    16#1F# => x"000A",
    16#20# => x"0000", -- HLT
    16#21# => x"0000",
    others  => (others => '0')
  );
end package;

# src/ram_sync.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;
use work.data_image_pkg.all;

entity ram_sync is
  port (
    clk_i  : in  std_logic;
    en_i   : in  std_logic;
    we_i   : in  std_logic;
    addr_i : in  addr_t;
    din_i  : in  word_t;
    dout_o : out word_t
  );
end entity;

architecture rtl of ram_sync is
  signal mem : ram256_t := RAM_INIT;
  signal q_r : word_t := (others => '0');
begin
  dout_o <= q_r;

  process(clk_i)
    variable idx : integer;
  begin
    if rising_edge(clk_i) then
      if en_i = '1' then
        idx := to_integer(unsigned(addr_i(7 downto 0)));
        if we_i = '1' then
          mem(idx) <= din_i;
        end if;
        q_r <= mem(idx);
      end if;
    end if;
  end process;
end architecture;

# src/reg_file12x16.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity reg_file12x16 is
  port (
    clk_i      : in  std_logic;
    rst_i      : in  std_logic;
    we_i       : in  std_logic;
    wr_addr_i  : in  reg_idx_t;
    rd_addr_a_i: in  reg_idx_t;
    rd_addr_b_i: in  reg_idx_t;
    din_i      : in  word_t;
    dout_a_o   : out word_t;
    dout_b_o   : out word_t
  );
end entity;

architecture rtl of reg_file12x16 is
  type reg_array_t is array (0 to REG_COUNT-1) of word_t;
  signal regs : reg_array_t := (others => (others => '0'));

  function read_reg(mem : reg_array_t; idx : reg_idx_t) return word_t is
  begin
    if to_integer(idx) < REG_COUNT then
      return mem(to_integer(idx));
    else
      return (others => '0');
    end if;
  end function;
begin
  dout_a_o <= read_reg(regs, rd_addr_a_i);
  dout_b_o <= read_reg(regs, rd_addr_b_i);

  process(clk_i)
  begin
    if rising_edge(clk_i) then
      if rst_i = '1' then
        regs <= (others => (others => '0'));
      elsif we_i = '1' and to_integer(wr_addr_i) < REG_COUNT then
        regs(to_integer(wr_addr_i)) <= din_i;
      end if;
    end if;
  end process;
end architecture;

# src/reg_file12x16_dbg.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity reg_file12x16_dbg is
  port (
    clk_i       : in  std_logic;
    rst_i       : in  std_logic;
    we_i        : in  std_logic;
    wr_addr_i   : in  reg_idx_t;
    rd_addr_a_i : in  reg_idx_t;
    rd_addr_b_i : in  reg_idx_t;
    din_i       : in  word_t;
    dout_a_o    : out word_t;
    dout_b_o    : out word_t;
    r0_o        : out word_t;
    r1_o        : out word_t;
    r2_o        : out word_t;
    r3_o        : out word_t;
    r4_o        : out word_t;
    r5_o        : out word_t;
    r6_o        : out word_t;
    r7_o        : out word_t;
    r8_o        : out word_t;
    r9_o        : out word_t;
    r10_o       : out word_t;
    r11_o       : out word_t
  );
end entity;

architecture rtl of reg_file12x16_dbg is
  type reg_array_t is array (0 to REG_COUNT-1) of word_t;
  signal regs : reg_array_t := (others => (others => '0'));

  function read_reg(mem : reg_array_t; idx : reg_idx_t) return word_t is
  begin
    if to_integer(idx) < REG_COUNT then
      return mem(to_integer(idx));
    else
      return (others => '0');
    end if;
  end function;
begin
  dout_a_o <= read_reg(regs, rd_addr_a_i);
  dout_b_o <= read_reg(regs, rd_addr_b_i);

  r0_o  <= regs(0);
  r1_o  <= regs(1);
  r2_o  <= regs(2);
  r3_o  <= regs(3);
  r4_o  <= regs(4);
  r5_o  <= regs(5);
  r6_o  <= regs(6);
  r7_o  <= regs(7);
  r8_o  <= regs(8);
  r9_o  <= regs(9);
  r10_o <= regs(10);
  r11_o <= regs(11);

  process(clk_i)
  begin
    if rising_edge(clk_i) then
      if rst_i = '1' then
        regs <= (others => (others => '0'));
      elsif we_i = '1' and to_integer(wr_addr_i) < REG_COUNT then
        regs(to_integer(wr_addr_i)) <= din_i;
      end if;
    end if;
  end process;
end architecture;

# src/rom_sync.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;
use work.program_image_pkg.all;

entity rom_sync is
  port (
    clk_i  : in  std_logic;
    en_i   : in  std_logic;
    addr_i : in  addr_t;
    data_o : out word_t
  );
end entity;

architecture rtl of rom_sync is
  signal q_r : word_t := (others => '0');
begin
  data_o <= q_r;

  process(clk_i)
    variable idx : integer;
  begin
    if rising_edge(clk_i) then
      if en_i = '1' then
        idx := to_integer(unsigned(addr_i(7 downto 0)));
        q_r <= ROM_INIT(idx);
      end if;
    end if;
  end process;
end architecture;

# src/stack7x16.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity stack7x16 is
  port (
    clk_i   : in  std_logic;
    rst_i   : in  std_logic;
    push_i  : in  std_logic;
    pop_i   : in  std_logic;
    din_i   : in  word_t;
    dout_o  : out word_t;
    sp_o    : out unsigned(2 downto 0);
    empty_o : out std_logic;
    full_o  : out std_logic
  );
end entity;

architecture rtl of stack7x16 is
  type stack_mem_t is array (0 to STACK_DEPTH-1) of word_t;
  signal mem    : stack_mem_t := (others => (others => '0'));
  signal sp_r   : unsigned(2 downto 0) := to_unsigned(STACK_DEPTH, 3);
  signal dout_r : word_t := (others => '0');
begin
  dout_o  <= dout_r;
  sp_o    <= sp_r;
  empty_o <= '1' when sp_r = to_unsigned(STACK_DEPTH, 3) else '0';
  full_o  <= '1' when sp_r = to_unsigned(0, 3) else '0';

  process(clk_i)
    variable sp_i : integer;
  begin
    if rising_edge(clk_i) then
      if rst_i = '1' then
        mem    <= (others => (others => '0'));
        sp_r   <= to_unsigned(STACK_DEPTH, 3);
        dout_r <= (others => '0');
      else
        sp_i := to_integer(sp_r);

        if push_i = '1' and pop_i = '0' then
          if sp_i > 0 then
            mem(sp_i - 1) <= din_i;
            sp_r <= to_unsigned(sp_i - 1, 3);
          end if;
        elsif pop_i = '1' and push_i = '0' then
          if sp_i < STACK_DEPTH then
            dout_r <= mem(sp_i);
            sp_r   <= to_unsigned(sp_i + 1, 3);
          end if;
        end if;
      end if;
    end if;
  end process;
end architecture;

# src/system_core.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity system_core is
  port (
    clk_i          : in  std_logic;
    rst_i          : in  std_logic;
    dma_start_i    : in  std_logic;
    dma_valid_i    : in  std_logic;
    dma_data_i     : in  word_t;

    halt_o         : out std_logic;
    dma_done_o     : out std_logic;

    dbg_state_o    : out std_logic_vector(4 downto 0);
    dbg_pc_o       : out addr_t;
    dbg_ir0_o      : out word_t;
    dbg_ir1_o      : out word_t;
    dbg_r1_o       : out word_t;
    dbg_r2_o       : out word_t;
    dbg_r3_o       : out word_t;
    dbg_r4_o       : out word_t;
    dbg_r5_o       : out word_t;
    dbg_r6_o       : out word_t;
    dbg_r7_o       : out word_t;
    dbg_flags_o    : out std_logic_vector(3 downto 0);
    dbg_sp_o       : out unsigned(2 downto 0);

    dbg_cache_hit_o  : out std_logic;
    dbg_cache_miss_o : out std_logic;
    dbg_req_cpu_o    : out std_logic;
    dbg_req_dma_o    : out std_logic;
    dbg_gnt_cpu_o    : out std_logic;
    dbg_gnt_dma_o    : out std_logic;
    dbg_ram_we_o     : out std_logic;
    dbg_ram_addr_o   : out addr_t;
    dbg_ram_wdata_o  : out word_t;
    dbg_ram_rdata_o  : out word_t;
    dbg_bp_hist_o    : out std_logic_vector(1 downto 0);
    dbg_bp_pred_o    : out std_logic;
    dbg_bp_target_o  : out addr_t
  );
end entity;

architecture structural of system_core is
  signal rom_addr     : addr_t;
  signal rom_en       : std_logic;
  signal rom_data     : word_t;

  signal cache_req    : std_logic;
  signal cache_we     : std_logic;
  signal cache_addr   : addr_t;
  signal cache_wdata  : word_t;
  signal cache_rdata  : word_t;
  signal cache_ready  : std_logic;
  signal cache_hit    : std_logic;
  signal cache_miss   : std_logic;

  signal cache_ram_req   : std_logic;
  signal cache_ram_we    : std_logic;
  signal cache_ram_addr  : addr_t;
  signal cache_ram_wdata : word_t;

  signal dma_req      : std_logic;
  signal dma_grant    : std_logic;
  signal cpu_grant    : std_logic;
  signal dma_done     : std_logic;
  signal dma_we       : std_logic;
  signal dma_addr     : addr_t;
  signal dma_wdata    : word_t;

  signal ram_en       : std_logic;
  signal ram_we       : std_logic;
  signal ram_addr     : addr_t;
  signal ram_wdata    : word_t;
  signal ram_rdata    : word_t;

  signal bp_query     : std_logic;
  signal bp_pc_query  : addr_t;
  signal bp_hist      : std_logic_vector(1 downto 0);
  signal bp_pred      : std_logic;
  signal bp_target    : addr_t;
  signal bp_update    : std_logic;
  signal bp_pc_update : addr_t;
  signal bp_hist_upd  : std_logic_vector(1 downto 0);
  signal bp_taken     : std_logic;
  signal bp_tgt_upd   : addr_t;
begin

ram_en    <= (cache_ram_req and cpu_grant) or (dma_we and dma_grant);
  ram_we    <= dma_we when dma_grant = '1' else cache_ram_we;
  ram_addr  <= dma_addr when dma_grant = '1' else cache_ram_addr;
  ram_wdata <= dma_wdata when dma_grant = '1' else cache_ram_wdata;

  dma_done_o        <= dma_done;
  dbg_cache_hit_o   <= cache_hit;
  dbg_cache_miss_o  <= cache_miss;
  dbg_req_cpu_o     <= cache_ram_req;
  dbg_req_dma_o     <= dma_req;
  dbg_gnt_cpu_o     <= cpu_grant;
  dbg_gnt_dma_o     <= dma_grant;
  dbg_ram_we_o      <= ram_we;
  dbg_ram_addr_o    <= ram_addr;
  dbg_ram_wdata_o   <= ram_wdata;
  dbg_ram_rdata_o   <= ram_rdata;
  dbg_bp_hist_o     <= bp_hist;
  dbg_bp_pred_o     <= bp_pred;
  dbg_bp_target_o   <= bp_target;

  U_CPU : entity work.cpu_core
    port map (
      clk_i          => clk_i,
      rst_i          => rst_i,
      rom_addr_o     => rom_addr,
      rom_en_o       => rom_en,
      rom_data_i     => rom_data,
      cache_req_o    => cache_req,
      cache_we_o     => cache_we,
      cache_addr_o   => cache_addr,
      cache_wdata_o  => cache_wdata,
      cache_rdata_i  => cache_rdata,
      cache_ready_i  => cache_ready,
      cache_hit_i    => cache_hit,
      cache_miss_i   => cache_miss,
      bp_query_o     => bp_query,
      bp_pc_query_o  => bp_pc_query,
      bp_hist_i      => bp_hist,
      bp_pred_i      => bp_pred,
      bp_target_i    => bp_target,
      bp_update_o    => bp_update,
      bp_pc_update_o => bp_pc_update,
      bp_hist_o      => bp_hist_upd,
      bp_taken_o     => bp_taken,
      bp_target_o    => bp_tgt_upd,
      halt_o         => halt_o,
      dbg_state_o    => dbg_state_o,
      dbg_pc_o       => dbg_pc_o,
      dbg_ir0_o      => dbg_ir0_o,
      dbg_ir1_o      => dbg_ir1_o,
      dbg_r1_o       => dbg_r1_o,
      dbg_r2_o       => dbg_r2_o,
      dbg_r3_o       => dbg_r3_o,
      dbg_r4_o       => dbg_r4_o,
      dbg_r5_o       => dbg_r5_o,
      dbg_r6_o       => dbg_r6_o,
      dbg_r7_o       => dbg_r7_o,
      dbg_flags_o    => dbg_flags_o,
      dbg_sp_o       => dbg_sp_o
    );

  U_ROM : entity work.rom_sync
    port map (
      clk_i  => clk_i,
      en_i   => rom_en,
      addr_i => rom_addr,
      data_o => rom_data
    );

  U_CACHE : entity work.cache_4way_age
    port map (
      clk_i       => clk_i,
      rst_i       => rst_i,
      cpu_req_i   => cache_req,
      cpu_we_i    => cache_we,
      cpu_addr_i  => cache_addr,
      cpu_wdata_i => cache_wdata,
      cpu_rdata_o => cache_rdata,
      cpu_ready_o => cache_ready,
      hit_o       => cache_hit,
      miss_o      => cache_miss,
      ram_req_o   => cache_ram_req,
      ram_we_o    => cache_ram_we,
      ram_addr_o  => cache_ram_addr,
      ram_wdata_o => cache_ram_wdata,
      ram_rdata_i => ram_rdata,

ram_grant_i => cpu_grant
    );

  U_RAM : entity work.ram_sync
    port map (
      clk_i  => clk_i,
      en_i   => ram_en,
      we_i   => ram_we,
      addr_i => ram_addr,
      din_i  => ram_wdata,
      dout_o => ram_rdata
    );

  U_ARB : entity work.bus_arbiter_2master
    port map (
      req_cpu_i   => cache_ram_req,
      req_dma_i   => dma_req,
      grant_cpu_o => cpu_grant,
      grant_dma_o => dma_grant
    );

  U_DMA : entity work.dma_controller_3word
    port map (
      clk_i       => clk_i,
      rst_i       => rst_i,
      start_i     => dma_start_i,
      grant_i     => dma_grant,
      dev_valid_i => dma_valid_i,
      dev_data_i  => dma_data_i,
      req_o       => dma_req,
      busy_o      => open,
      done_o      => dma_done,
      ram_we_o    => dma_we,
      ram_addr_o  => dma_addr,
      ram_data_o  => dma_wdata
    );

  U_BP : entity work.branch_predictor
    port map (
      clk_i          => clk_i,
      rst_i          => rst_i,
      query_valid_i  => bp_query,
      pc_query_i     => bp_pc_query,
      hist_o         => bp_hist,
      pred_taken_o   => bp_pred,
      pred_target_o  => bp_target,
      update_valid_i => bp_update,
      pc_update_i    => bp_pc_update,
      hist_i         => bp_hist_upd,
      actual_taken_i => bp_taken,
      actual_tgt_i   => bp_tgt_upd
    );
end architecture;

# tb/tb_alu_core.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity tb_alu_core is
end entity;

architecture sim of tb_alu_core is
  signal a      : word_t := (others => '0');
  signal b      : word_t := (others => '0');
  signal sflag  : std_logic := '0';
  signal op     : std_logic_vector(2 downto 0) := ALU_PASS_A;
  signal y      : word_t;
  signal z, s, c, o : std_logic;
begin
  DUT : entity work.alu_core
    port map (
      a_i => a,
      b_i => b,
      flag_s_i => sflag,
      op_i => op,
      y_o => y,
      z_o => z,
      s_o => s,
      c_o => c,
      o_o => o
    );

  stimulus : process
  begin
    a <= x"8001";
    b <= x"00F0";
    op <= ALU_SRA;
    wait for 10 ns;
    assert y = x"C000" and c = '1' and s = '1'
      report "SRA 8001h must produce C000h, C=1, S=1" severity failure;

    a <= y;
    sflag <= '1';
    op <= ALU_INCS;
    wait for 10 ns;
    assert y = x"C001"
      report "INCS with S=1 must increment C000h to C001h" severity failure;

    a <= x"00F0";
    b <= x"0F0F";
    op <= ALU_OR;
    wait for 10 ns;
    assert y = x"0FFF"
      report "OR must produce 0FFFh" severity failure;

    a <= x"0FFF";
    b <= x"0FFF";
    op <= ALU_NOR;
    wait for 10 ns;
    assert y = x"F000"
      report "NOR must produce F000h" severity failure;

    sflag <= '0';
    a <= x"1234";
    op <= ALU_INCS;
    wait for 10 ns;
    assert y = x"1234"
      report "INCS with S=0 must keep source value" severity failure;

    assert false report "tb_alu_core: TEST PASSED" severity note;
    wait;
  end process;
end architecture;

# tb/tb_mem_dma.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity tb_mem_dma is
end entity;

architecture sim of tb_mem_dma is
  signal clk      : std_logic := '0';
  signal rst      : std_logic := '1';
  signal start    : std_logic := '0';
  signal dev_valid: std_logic := '0';
  signal dev_data : word_t    := (others => '0');
  signal dma_req  : std_logic;
  signal dma_gnt  : std_logic;
  signal dma_done : std_logic;
  signal ram_we   : std_logic;
  signal ram_addr : addr_t;
  signal ram_din  : word_t;
  signal ram_dout : word_t;
begin
  clk <= not clk after 5 ns;
  dma_gnt <= dma_req;

  U_DMA : entity work.dma_controller_3word
    port map (
      clk_i => clk,
      rst_i => rst,
      start_i => start,
      grant_i => dma_gnt,
      dev_valid_i => dev_valid,
      dev_data_i => dev_data,
      req_o => dma_req,
      busy_o => open,
      done_o => dma_done,
      ram_we_o => ram_we,
      ram_addr_o => ram_addr,
      ram_data_o => ram_din
    );

  U_RAM : entity work.ram_sync
    port map (
      clk_i => clk,
      en_i => '1',
      we_i => ram_we,
      addr_i => ram_addr,
      din_i => ram_din,
      dout_o => ram_dout
    );

  monitor : process(clk)
    variable write_count : integer := 0;
  begin
    if rising_edge(clk) then
      if rst = '1' then
        write_count := 0;
      elsif ram_we = '1' then
        case write_count is
          when 0 =>
            assert ram_addr = x"000A" and ram_din = x"1111"
              report "DMA first write must target 000Ah with 1111h" severity failure;
          when 1 =>
            assert ram_addr = x"000B" and ram_din = x"2222"
              report "DMA second write must target 000Bh with 2222h" severity failure;
          when 2 =>
            assert ram_addr = x"000C" and ram_din = x"3333"
              report "DMA third write must target 000Ch with 3333h" severity failure;
          when others =>
            assert false report "Unexpected extra DMA write" severity failure;
        end case;
        write_count := write_count + 1;
      end if;
    end if;
  end process;

  stimulus : process
  begin
    wait for 30 ns;
    rst <= '0';

    wait until rising_edge(clk);
    start    <= '1';
    dev_valid <= '1';
    dev_data <= x"1111";
    wait until rising_edge(clk);
    start <= '0';

    wait until rising_edge(clk);
    dev_data <= x"1111";
    wait until rising_edge(clk);
    dev_data <= x"2222";
    wait until rising_edge(clk);
    dev_data <= x"3333";
    wait until rising_edge(clk);
    dev_valid <= '0';
    wait until rising_edge(clk);
    wait for 1 ns;
    assert dma_done = '1'
      report "DMA must assert done after three words" severity failure;

    assert false report "tb_mem_dma: TEST PASSED" severity note;
    wait;

end process;
end architecture;

# tb/tb_stack7x16.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity tb_stack7x16 is
end entity;

architecture sim of tb_stack7x16 is
  signal clk   : std_logic := '0';
  signal rst   : std_logic := '1';
  signal push  : std_logic := '0';
  signal pop   : std_logic := '0';
  signal din   : word_t := (others => '0');
  signal dout  : word_t;
  signal sp    : unsigned(2 downto 0);
  signal empty : std_logic;
  signal full  : std_logic;
begin
  clk <= not clk after 5 ns;

  DUT : entity work.stack7x16
    port map (
      clk_i => clk,
      rst_i => rst,
      push_i => push,
      pop_i => pop,
      din_i => din,
      dout_o => dout,
      sp_o => sp,
      empty_o => empty,
      full_o => full
    );

  stimulus : process
  begin
    wait for 12 ns;
    rst <= '0';
    wait for 1 ns;
    assert sp = to_unsigned(7, 3) and empty = '1' and full = '0'
      report "After reset stack must be empty with SP=7" severity failure;

    din <= x"0002"; push <= '1'; wait for 10 ns; push <= '0';
    din <= x"0006"; push <= '1'; wait for 10 ns; push <= '0';
    din <= x"0007"; push <= '1'; wait for 10 ns; push <= '0';
    din <= x"0078"; push <= '1'; wait for 10 ns; push <= '0';
    din <= x"03E0"; push <= '1'; wait for 10 ns; push <= '0';
    wait for 1 ns;
    assert sp = to_unsigned(2, 3)
      report "After five pushes SP must be 2" severity failure;

    pop <= '1'; wait for 10 ns; pop <= '0';
    wait for 1 ns;
    assert dout = x"03E0"
      report "First pop must return last pushed value 03E0h" severity failure;
    pop <= '1'; wait for 10 ns; pop <= '0';
    pop <= '1'; wait for 10 ns; pop <= '0';
    pop <= '1'; wait for 10 ns; pop <= '0';
    pop <= '1'; wait for 10 ns; pop <= '0';
    wait for 1 ns;
    assert sp = to_unsigned(7, 3) and empty = '1'
      report "After five pops stack must be empty with SP=7" severity failure;

    assert false report "tb_stack7x16: TEST PASSED" severity note;
    wait;
  end process;
end architecture;

# tb/tb_system.vhd

library ieee;
use ieee.std_logic_1164.all;
use ieee.numeric_std.all;
use work.microcomputer_pkg.all;

entity tb_system is
end entity;

architecture sim of tb_system is
  signal clk          : std_logic := '0';
  signal rst          : std_logic := '1';
  signal dma_start    : std_logic := '0';
  signal dma_valid    : std_logic := '0';
  signal dma_data     : word_t := (others => '0');
  signal halt         : std_logic;
  signal dma_done     : std_logic;

  signal dbg_state    : std_logic_vector(4 downto 0);
  signal dbg_pc       : addr_t;
  signal dbg_ir0      : word_t;
  signal dbg_ir1      : word_t;
  signal dbg_r1       : word_t;
  signal dbg_r2       : word_t;
  signal dbg_r3       : word_t;
  signal dbg_r4       : word_t;
  signal dbg_r5       : word_t;
  signal dbg_r6       : word_t;
  signal dbg_r7       : word_t;
  signal dbg_flags    : std_logic_vector(3 downto 0);
  signal dbg_sp       : unsigned(2 downto 0);
  signal cache_hit    : std_logic;
  signal cache_miss   : std_logic;
  signal req_cpu      : std_logic;
  signal req_dma      : std_logic;
  signal gnt_cpu      : std_logic;
  signal gnt_dma      : std_logic;
  signal ram_we       : std_logic;
  signal ram_addr     : addr_t;
  signal ram_wdata    : word_t;
  signal ram_rdata    : word_t;
  signal bp_hist      : std_logic_vector(1 downto 0);
  signal bp_pred      : std_logic;
  signal bp_target    : addr_t;
begin
  clk <= not clk after 5 ns;

  UUT : entity work.system_core
    port map (
      clk_i            => clk,
      rst_i            => rst,
      dma_start_i      => dma_start,
      dma_valid_i      => dma_valid,
      dma_data_i       => dma_data,
      halt_o           => halt,
      dma_done_o       => dma_done,
      dbg_state_o      => dbg_state,
      dbg_pc_o         => dbg_pc,
      dbg_ir0_o        => dbg_ir0,
      dbg_ir1_o        => dbg_ir1,
      dbg_r1_o         => dbg_r1,
      dbg_r2_o         => dbg_r2,
      dbg_r3_o         => dbg_r3,
      dbg_r4_o         => dbg_r4,
      dbg_r5_o         => dbg_r5,
      dbg_r6_o         => dbg_r6,
      dbg_r7_o         => dbg_r7,
      dbg_flags_o      => dbg_flags,
      dbg_sp_o         => dbg_sp,
      dbg_cache_hit_o  => cache_hit,
      dbg_cache_miss_o => cache_miss,
      dbg_req_cpu_o    => req_cpu,
      dbg_req_dma_o    => req_dma,
      dbg_gnt_cpu_o    => gnt_cpu,
      dbg_gnt_dma_o    => gnt_dma,
      dbg_ram_we_o     => ram_we,
      dbg_ram_addr_o   => ram_addr,
      dbg_ram_wdata_o  => ram_wdata,
      dbg_ram_rdata_o  => ram_rdata,
      dbg_bp_hist_o    => bp_hist,
      dbg_bp_pred_o    => bp_pred,
      dbg_bp_target_o  => bp_target
    );

  stim : process
  begin
    rst <= '1';
    wait for 30 ns;
    rst <= '0';

    wait until rising_edge(clk);
    wait until rising_edge(clk);

dma_start <= '1';
    dma_valid <= '1';
    dma_data  <= x"1111";
    wait until rising_edge(clk);
    dma_start <= '0';

    wait until rising_edge(clk);
    dma_data <= x"1111";
    wait until rising_edge(clk);
    dma_data <= x"2222";
    wait until rising_edge(clk);
    dma_data <= x"3333";
    wait until rising_edge(clk);
    dma_valid <= '0';

    wait until halt = '1';
    wait for 20 ns;

    assert dbg_r1 = x"C001" report "R1 must be C001 after SRA + INCS" severity failure;
    assert dbg_r2 = x"F000" report "R2 must be F000 after OR + NOR" severity failure;
    assert dbg_r3 = x"C001" report "R3 must receive popped stack value" severity failure;
    assert dbg_r4 = x"0000" report "R4 must be zero after NOR FFFF,FFFF" severity failure;
    assert dbg_r5 = x"0000" report "R5 must remain zero because JZ skips 0018h" severity failure;
    assert dbg_r6 = x"0000" report "R6 must remain zero because JMP skips 001Ch" severity failure;
    assert dbg_r7 = x"1111" report "R7 must read first DMA word from RAM[000Ah]" severity failure;
    assert dbg_flags = "1000" report "Final flags must be Z=1, S=0, C=0, O=0" severity failure;
    assert dbg_sp = to_unsigned(7, 3) report "Stack pointer must return to empty value 7" severity failure;
    assert dma_done = '1' report "DMA must complete three-word transfer" severity failure;
    assert bp_hist = "01" report "GHR must become 01 after one taken JZ" severity failure;

    assert false report "tb_system: TEST PASSED" severity note;
    wait;
  end process;
end architecture;